# Blockchain-Based Identity and Provenance for Distributed Energy Assets

Immutable Identity, Lifecycle Traceability, and Integrity Verification for Solar and Renewable Infrastructure

[AUTHOR NAME — TO BE SUPPLIED]

Table of Contents

# Structural Defect Mapping and Integrity Verification

## What This Chapter Covers

This is the book’s anchor chapter, and the one that draws most directly on my own research. It develops the passive-binding branch of Chapter 3 to its strongest form: mapping the defect structure of photovoltaic devices, binding those maps to the ledger identity record at enrollment, and building on that binding a verification workflow through which a downstream party — an insurer, a buyer, a grid operator, a warranty adjudicator — can trust a panel’s recorded condition history without trusting its custodian. Everything Part II has built converges here: the enrollment moment of Chapter 4 gets its measurement, the EVT_REGISTER and EVT_INSPECT events of Chapter 5 get their payloads, and the four identity properties of Chapter 1 finally get their physical root. Portions of the mechanism described in Sections 6.4–6.6 are the subject of my pending patent application; as promised in the Preface, the ideas are explained fully, and readers evaluating alternative integrity-verification mechanisms will find the workflow of Section 6.6 transfers to any binding modality that satisfies the requirements of Section 6.5. Section 6.9 states the intellectual-property posture plainly so that no reader has to infer it from omissions.

Section 6.0 fixes terminology before the physics: this chapter is about structural defect mapping, not a survey of quantum technologies. Section 6.8 collects the open problems rather than burying them. The chapter runs from the physical argument (why defect structure is the right anchor, and why simpler marks are not), through the sensing toolbox with maturity stated plainly, the enrollment pipeline and its two-layer decomposition, the three-element binding construction, the generalized requirements for readers bringing other modalities, the four-step verification workflow with its sampling arithmetic, and the tier economics — closing with the open problems and, in Section 6.9, a plain statement of the intellectual-property posture so that readers can weigh the argument knowing their author’s stake.

## 6.0 Terminology: Structural Mapping, Quantum Modalities, and Classical Characterization

Measurement scientists and energy engineers use overlapping vocabulary for different instrument classes. This book uses the terms below consistently; a reviewer from either community should find no hidden category error.

Structural defect mapping is the chapter’s subject: recovering a per-unit fingerprint of the microscopic and mesoscopic defect population — grain boundaries, shunt paths, solder-bond resistance outliers, crack precursors — that manufacturing does not control and that evolves as the module degrades. The map is the identity anchor; the instruments are means to read it.

Quantum-limited sensing modalities are those whose readout exploits quantum states of a probe — spin ensembles, superconducting loops — to reach field or current sensitivity below what classical magnetometers achieve at comparable spatial resolution. In this architecture, nitrogen-vacancy (NV) magnetometry and laboratory scanning SQUID systems fall in this class. They are the high-assurance enrollment and escalation tools of Table 6.1’s upper tiers.

Established semiconductor characterization covers modalities the PV industry already deploys at factory and field scale: electroluminescence (EL) imaging, lock-in thermography, infrared thermography, and I–V characterization. These are not “quantum sensing” in the physics sense. They are mature, classical measurements of recombination, dissipation, and integral electrical behavior. In this architecture they provide corroboration, routine tiers, and fleet screening — not because they are inferior science, but because their economics and deployment maturity match high-volume manufacture.

Composite enrollment combines both classes: factory EL plus flash data at Tier 0 (cheap, universal), magnetometric defect maps at Tier 2 (higher cost, higher bindability), with cross-modal locking so neither modality alone carries the security argument. The patent-pending contribution described in Sections 6.4–6.6 concerns the binding construction and verification workflow across these modalities, not the claim that every measurement in the stack is quantum physics.

Readers arriving from quantum-metrology laboratories should expect EL and thermography to carry most of the transaction volume. Readers arriving from PV quality assurance should expect NV magnetometry to appear where bindability, not throughput, is the binding constraint. Both are correct.

## 6.1 Why Defects, of All Things

Chapter 3 established the requirements for a passive structural fingerprint: per-unit uniqueness, multi-decade stability of some feature subset, discrimination under field measurement noise, and forgery cost exceeding forgery value. Most proposals in this space begin from an instrument and search for an application; the argument here runs the other way — from the requirements to the physical layer that best satisfies them, and only then to the instruments that can read that layer. The defect structure of a crystalline silicon device is a strong candidate against all four, for a reason worth stating carefully:

The defect distribution is precisely what manufacturing does not control. A production line controls means and tolerances — cell efficiency binning, layer thicknesses, contact resistances. It does not, and cannot, control the microscopic placement of grain boundaries, dislocation clusters, impurity precipitates, and shunt paths; these arise from the stochastic physics of ingot solidification and cell processing. Two cells from adjacent wafer positions differ measurably and irreproducibly. For an attacker, this is the worst possible situation: forging a fingerprint means reproducing, in semiconductor bulk, a specific random pattern that the legitimate manufacturer could not reproduce with the same fab on the same day. The forgery-cost asymmetry of Section 3.4 is at its maximum here.

A short inventory of the defect population makes the argument concrete. In multicrystalline silicon, grain boundaries — the interfaces between randomly oriented crystal domains — form a per-wafer pattern set by the stochastic nucleation of the ingot’s solidification front; no two wafers share one, and the pattern is legible in both recombination imaging and current-path structure. Dislocation clusters — tangles of lattice defects that propagate during crystal growth — dot each wafer in densities and positions that vary even between adjacent wafers from one brick. In monocrystalline material, where grain structure is absent, individuality persists in subtler channels: oxygen precipitates and ring patterns from the crystal pull, striations from dopant fluctuation, and the statistical scatter of process-induced features — micro-shunts where emitter diffusion or edge isolation varied, metallization finger discontinuities from screen-printing stochastics, firing-profile signatures in contact resistance. Cell interconnection then adds the solder-bond population, each joint’s resistance its own small random variable. The population is layered, three-dimensional, and — decisively — sub-visual: most of it produces no optical contrast at the module surface, so the counterfeiter examining a module cannot even read the features he would need to reproduce without laboratory instrumentation.

### 6.1.1 Why Not Simpler Marks

Two engineered alternatives deserve dispatch before the physics, because both are regularly proposed as cheaper roads to the same destination. Laser-etched serial marks — engraving the identifier into the glass or cell — improve on labels (the mark survives weathering and cannot fall off) but remain identification, not authentication: the mark is applied to the object rather than being of it, so the forger’s task is unchanged — apply the same mark to a different object. Etching also cannot be re-issued without visible accumulation, and cell-level marks cost yield. Engineered taggants — DNA markers, rare-earth pigments, or micro-particle codes mixed into encapsulant or backsheet — genuinely bind to the material and have real anti-counterfeit value at the batch level (is this genuine manufacturer material?), but they are common to every unit sharing the material lot, so they cannot distinguish units; per-unit taggant dosing at module economics has not been demonstrated; and taggants are secrets — their security degrades as the formulation circulates, the opposite of a structural fingerprint whose “secret” is a random pattern nobody, including the manufacturer, chose or can reproduce. Both alternatives fail the same clause of the requirements: they add an identity artifact where the design needs an identity property. The defect map is the only candidate on the table that the object cannot be separated from, because it is the object.

Defects are also — and this is the design insight the rest of the chapter exploits — the same features that determine the asset’s condition and future. A fingerprint built on encapsulant speckle identifies the module but says nothing about it. A fingerprint built on the defect map identifies the module and is itself the baseline condition record: degradation is, physically, the evolution of this very map. Cracks propagate from stress concentrations the map recorded; PID develops preferentially where shunt precursors sat; solder-bond fatigue grows from the resistance outliers the enrollment measurement flagged. Identity verification and condition assessment collapse into one measurement, which is what makes the economics of Section 6.7 close — and, as a bonus the warranty industry will appreciate, the enrollment map is predictive: the defect population at manufacture is among the best available predictors of the module’s degradation trajectory, so the identity record doubles as an underwriting input (Chapter 13 returns to this).

## 6.2 The Sensing Toolbox

Table 6.1 organizes the modalities by role. This section states what each measures and where it fits; the physics is developed only to the depth the system architecture needs. Quantum-limited instruments appear first; classical characterization modalities follow in Section 6.2’s closing paragraphs and in Table 6.1’s lower rows.

Nitrogen-vacancy (NV) magnetometry. The quantum-limited workhorse, and worth a careful paragraph of physics because the architecture’s claims rest on what it can and cannot measure. An NV center is a point defect in diamond — a substitutional nitrogen atom adjacent to a lattice vacancy — whose ground state is a spin triplet with a convenient pair of properties: the spin can be initialized and read out optically (green excitation; the red fluorescence intensity depends on the spin state), and its energy sublevels shift linearly with the local magnetic field through the Zeeman effect. Sweeping a microwave drive across the spin resonance while monitoring fluorescence — optically detected magnetic resonance, ODMR — turns each NV center into an atomic-scale magnetometer that operates at room temperature, with no cryogenics and no shielding requirements beyond ordinary lab practice. Two deployment formats matter here. In wide-field imaging, a thin layer of NV centers engineered into a diamond plate is placed against (or within a millimeter of) the device; a camera reads the fluorescence of millions of centers in parallel, producing a full-field magnetic image in seconds to minutes at micrometer-scale resolution over centimeter-scale fields of view — tiled across a cell or module by scanning. In standoff arrays, discrete NV (or fluxgate) sensors at centimeter distances trade resolution for speed and robustness, suitable for field screening. Sensitivities depend on integration time and sensor engineering, spanning roughly the nanotesla to sub-nanotesla range in practical wide-field configurations — comfortably sufficient, since the fields from ampere-scale currents in cell metallization at millimeter distances sit in the microtesla regime.

The measured quantity is the magnetic field above the device; the wanted quantity is the current density within it, recovered by inverting the Biot–Savart relation. For the quasi-two-dimensional current sheets of a solar cell the inversion is well-conditioned up to a spatial-frequency limit set by the sensor standoff distance — the physics behind Table 6.1’s resolution-versus-deployment trade — and its regularization choices are part of the signed processing chain of Section 6.3 (and the subject of open problem M2). What current-density maps reveal that optical modalities cannot: shunt currents (localized vertical leakage paths), current detours around cracked or interrupted fingers, asymmetric injection from degraded solder bonds, and inactive regions — including variants of each that lie beneath the radiative-recombination layer an EL camera images, or that manifest only as current redistribution with no emission contrast at all. Magnetometry and EL are thus genuinely complementary at the physics level, which is what gives the cross-modal locking of Section 6.4 its teeth.

Scanning SQUID and fluxgate magnetometry. Higher field sensitivity (SQUIDs) at the cost of cryogenics and slow scanning — laboratory reference tools, not line tools. Fluxgate arrays offer a cheap, coarse intermediate useful for string-level field screening.

Field operating requirements for the magnetometric modalities deserve honest listing, since site conditions are not a laboratory. The measurement needs current through the device — forward bias injection (the EL condition, typically performed at night or under opaque covers for installed modules) or, in a useful daytime variant, the module’s own photocurrent under load modulation, which trades signal control for zero setup. It needs standoff control at millimeters for high-resolution work, achievable with contact fixtures on accessible modules and genuinely awkward on trackers at height — one reason the V3 tier often prefers pulling sampled modules to a ground station. It tolerates Earth’s field and site magnetic clutter through gradiometric and lock-in techniques (modulating the bias and detecting synchronously rejects static backgrounds), but high-current DC cabling nearby sets a noise floor the campaign plan must respect. None of these constraints is disqualifying; all of them cost minutes per unit, which is exactly why the sampling arithmetic of Section 6.6.1, rather than instrument physics, is what makes field verification affordable.

Complementary classical modalities. The system design that follows never relies on a single instrument class. EL imaging (fast, mature, optical-depth-limited), IR thermography (coarse, field-deployable), and I–V characterization (integral, not spatial) each corroborate the magnetometric map per Section 4.5’s Rule 2. These are established semiconductor-characterization tools, not quantum probes; they carry the routine tiers because their marginal cost and line speed are solved problems. Table 6.1 positions all modalities together.

Table 6.1 Measurement modalities for module structural characterization, as used in this chapter’s architecture. Speeds and maturities are stated for 2026 and will age in the reader’s favor; the roles column is the durable content.

| Modality | Measures | Resolution | Speed (60-cell module) | Deployment | Role here |
| --- | --- | --- | --- | --- | --- |
| NV magnetometry (near-contact) | Current density via field map | ~10 µm–1 mm | Minutes (bench, current systems) | Lab / factory line (emerging) | Enrollment fingerprint; escalated verification |
| NV magnetometry (standoff array) | Coarse current anomalies | ~mm–cm | Seconds–minutes | Field-portable (prototype) | Field re-verification |
| Scanning SQUID | Current density | ~µm | Hours | Cryogenic lab | Reference / calibration |
| EL imaging | Radiative recombination pattern | ~100 µm–1 mm | Seconds | Factory + field (mature) | Corroborating fingerprint; routine checks |
| IR thermography | Dissipative hot spots | ~cm | Seconds (drone) | Field (mature) | Fleet screening, triage |
| I–V / dark I–V | Integral electrical params | none (integral) | Seconds | Factory + field (mature) | Cheap corroboration |

The complementary classical modalities deserve their own cost-and-capability honesty, since they carry the routine tiers. Factory EL is a solved problem: in-line EL stations are standard equipment on modern lines, cycle within takt, and produce full-module images whose per-unit marginal cost is electricity and storage. Field EL has matured rapidly — daylight-capable contact rigs and drone-mounted nighttime EL both operate commercially — with per-module costs in the single dollars at campaign scale. Drone IR thermography is cheaper still per module (entire plants in days) at the price of coarse, screening-grade information. Flash testers and portable I–V tracers are ubiquitous. The quantum layer’s instruments, by contrast, currently price like laboratory equipment: wide-field NV microscopes are six-figure instruments built in small volumes, and a factory-hardened line-speed variant is a product that does not yet exist. The architecture’s wager — argued in Section 6.7’s tiering rather than assumed — is that this cost curve follows the usual trajectory of sensing instruments once an industrial demand exists, and that in the meantime the small installed base serves the escalation tiers, where a six-figure instrument amortized across adjudications and acquisitions is cheap.

Every instrument in the table is also, per Section 4.5’s Rule 3, an asset in the system: the defect-map scanner that enrolls a million modules is a million-fold single point of trust, and its own identity, firmware attestation, and calibration lifecycle receive I-A treatment (Table 4.3) without exception. The Tier 3 reference laboratory’s role is to anchor this recursion: field and factory instruments calibrate against transported reference artifacts — modules with laboratory-grade characterization whose own histories live on the ledger — so that a template’s meaning is traceable through the instrument chain to a metrological root, in exactly the pattern dimensional and electrical metrology have used for a century.

The honest maturity statement: factory-speed NV magnetometry of full modules is at the advanced-prototype stage — bench systems image cells in minutes; in-line throughput at seconds-per-module is an engineering extrapolation with identified paths (wide-field NV imaging, parallel sensor tiles) rather than a shipping product. The architecture is therefore designed so that EL carries the routine load today and the magnetometric layer raises the assurance ceiling where and when it is deployed — a deployment-time dial, not an all-or-nothing bet (Section 6.7).

## 6.3 From Measurement to Fingerprint Template

A raw defect map is a large image-like object (Table 4.1 budgeted 10–100 MB); a fingerprint template is the compact, comparison-ready derivative committed at enrollment. The distinction matters legally as well as technically: the raw map is the evidence, preserved in custody for the disputes that want everything; the template is the operational instrument, sized and structured for millions of fast comparisons; and because each processing stage signs its output against its input’s digest, a court or counterparty can always walk from the operational answer back to the primary evidence and recompute the walk. The pipeline, each stage signed per the provenance-chain discipline of Section 4.5:

- Registration and normalization. Map is registered to the module’s cell grid (busbars and cell edges provide the frame), normalized for bias current and temperature — both recorded, since comparison across operating points is a known error source. The error budget assigns this stage its own allocation: registration residuals must sit well below the smallest stable feature’s scale, which the busbar skeleton achieves comfortably; temperature normalization matters because shunt behavior and series resistances are temperature-dependent, and the enrollment record therefore stores the measurement conditions as first-class data rather than as lab-notebook afterthoughts.

- Feature decomposition. The map is decomposed into a stable-structure layer (grain-boundary current texture, as-built shunt population, fixed process signatures) and a condition layer (features known to evolve: crack networks, solder-bond resistances, PID-susceptible patterns). The decomposition is the scientific heart of the scheme — Section 3.4 previewed why: identity must rest on the stable layer, while the condition layer becomes the degradation record. The assignment of feature classes to layers is itself versioned scientific content: it encodes the field’s current understanding of which features age, it improves as longitudinal data accumulates (open problem M1), and the pilot’s mid-course re-decomposition (Section 11.5, lesson 6) was precisely an upgrade of this assignment, executed through the schema’s versioning machinery rather than through a data migration.

- Template encoding. The stable layer is encoded as a feature vector with a defined similarity metric and decision thresholds calibrated on false-accept / false-reject trade-offs (the biometric formalism transfers directly, and Appendix B’s comparison table uses its vocabulary). The condition layer is encoded as a versioned condition summary. Threshold calibration is empirical and cohort-based: the FAR side is estimated by cross-comparing templates across large production populations (every pair of distinct modules is a labeled impostor trial, so a production month yields billions of trials free), while the FRR side requires re-measurement campaigns under field conditions — scarcer data, and the reason verification protocols always carry a re-measure-before-escalate step.

- Commitment. Template digest and condition-summary digest enter the EVT_REGISTER (or EVT_INSPECT) payload; full maps go to replicated off-chain custody per Section 4.2, encrypted per Section 4.2.1 — the full map is commercially sensitive (it reveals process signatures a competitor could read) even though the template digest reveals nothing.

Template sizes, for the record and for Chapter 7’s storage arithmetic: the stable-structure template encodes to tens or hundreds of kilobytes per module depending on modality and version (grain-texture features dominate); the condition summary is similar; the underlying full maps are the 10–100 MB payloads of Table 4.1. Only digests of any of these touch the ledger. The encoding is a versioned, documented format per Section 4.2.1’s longevity rules — and its openness is precisely what standardization gap S3 is about, since a template only proves anything to a verifier whose software can compute the same features from a fresh measurement.

Figure 6.1 The enrollment pipeline. Everything to the right of the instrument is a signed computation chain; the two-layer decomposition feeds identity and condition records separately.

flowchart LR
    M[Module under bias] --> Q[NV magnetometer<br>signed raw field map]
    Q --> INV[Current-density<br>inversion, signed]
    INV --> REG[Grid registration &<br>normalization, signed]
    REG --> DEC{Two-layer<br>decomposition}
    DEC --> ST[Stable-structure layer<br>→ identity template]
    DEC --> CN[Condition layer<br>→ condition summary]
    ST --> TD[Template digest]
    CN --> CD[Condition digest]
    TD --> EV[EVT_REGISTER /<br>EVT_INSPECT payload]
    CD --> EV
    Q -. full maps .-> STOR[(Replicated<br>off-chain custody)]
    EV --> LEDG[Consortium ledger<br>+ public anchor]

## 6.4 Binding the Map to the Identity Record

The binding construction — the part of the mechanism at the center of the patent application — has three elements beyond the pipeline above.

Cross-modal locking. The enrollment event commits templates from at least two physically independent modalities (magnetometric stable-structure plus EL structural) with a recorded spatial co-registration between them. An attacker must now forge two physically distinct signatures and their mutual geometric relationship — and the co-registration is cheap for the legitimate enroller (one fixture, one timestamp) while roughly squaring the forger’s problem, since the modalities sample different depths of the device (EL sees radiative recombination; magnetometry sees current paths, including buried ones). Mechanically, co-registration is a shared coordinate solution: both instruments image the same fiducial skeleton (busbars, cell edges) in the same fixture without re-handling the module between captures, and the enrollment payload records the transform between the two feature sets along with its residuals. At verification, the lock is checked by confirming not only that each modality matches its template but that features shared across modalities — a grain boundary visible in EL whose current-texture counterpart appears in the field map — sit at the recorded mutual positions. A forger who solves each modality separately (already implausible) discovers that the solutions must also be the same object’s solutions, aligned to sub-feature precision. Independence between the modalities is what makes the multiplication legitimate; this is Rule 2 of Section 4.5 executed in physics rather than in signatures.

Challenge-parameterized measurement. The field verification protocol does not simply re-measure and compare. The verifier draws a random challenge — a bias current level and a subset of cells/regions, from a challenge space fixed at enrollment — and the comparison is performed on the challenged slice. Because current-flow patterns vary with bias point in a device-specific, defect-determined way, a static replica (a “defect decal”) that matches one operating point fails at another. This imports the liveness logic of challenge–response authentication (Section 3.3) into a device with no processor: the physics answers the challenge.

The challenge space’s size is what makes the mechanism more than a gesture. Enrollment captures the map at several bias points spanning the device’s operating range, from which the bias response of each stable feature is characterized — shunts whose current share grows nonlinearly with bias, detour paths whose visibility switches with injection level, resistive signatures that migrate. A challenge then names a bias point (drawn from a continuum, not a menu), a cell subset, and a feature class; the expected response is computable from the enrollment characterization but the full response surface was never published — only its digest-committed parameterization, disclosed slice by slice as challenges consume it. An adversary who obtained every template in the registry still faces the question a static artifact cannot answer: how does your defect population respond to a bias point nobody has asked about before? Only a device with an actual, physically consistent defect population responds correctly, and manufacturing such a device is the R3 problem again. The design’s honest caveat is inversion ambiguity — whether distinct defect populations could share response surfaces within measurement error across the challenge space — which is exactly open problem M2, stated so the research community can attack it rather than discover it.

Supersession with provenance. Structure evolves; templates age. EVT_REENROLL (Section 5.3) lets an accredited verifier commit a new template that supersedes the old with an explicit reason and a full measurement trail — never replacing it. The chain of superseded templates is itself evidence: each re-enrollment’s condition layer must be a physically plausible evolution of its predecessor (cracks extend, they do not heal), and implausible transitions flag substitution. Identity, on this design, is not a static match to a birth certificate but continuity of documented physical evolution — which is, on reflection, how identity works for every aging physical thing, formalized.

The evolution-plausibility check (requirement R5) deserves its mechanics spelled out, because it is the supersession mechanism’s immune system. Crack networks grow monotonically: a verification image whose crack set is a strict superset of the enrollment’s, with extensions rooted at recorded stress concentrations, is a plausible successor; an image whose cracks have vanished, migrated, or appeared with morphologies inconsistent with the module’s recorded mechanical history is not. Shunt populations drift in bounded ways — existing shunts strengthen with PID stress and partially recover under reverse-bias regimes, but new shunts do not materialize in patterns uncorrelated with cell structure. Solder-bond resistances walk upward with thermal-cycle counts at rates the fatigue literature bounds. Each of these is a checkable directional constraint, and the checks compose: a substituted module must not merely resemble the target — it must present a condition layer that is a physically legal evolution of a specific recorded past it never lived. The check’s power grows with every recorded inspection (more waypoints to be consistent with), which pleasingly means the assets with the most valuable histories are also the hardest to impersonate.

## 6.5 Requirements Stated Generally

For readers who will adopt a different sensing modality — and to keep the architecture honest about what it actually depends on — the properties the binding layer requires of any fingerprint mechanism:

Table 6.2 Requirements on a passive binding modality, and how the defect-map construction meets them.

| Requirement | General statement | Defect-map realization |
| --- | --- | --- |
| R1 Uniqueness entropy | Feature space large and device-individual | Stochastic solidification/process defects |
| R2 Stable subset | Identifiable features invariant over service life at field measurement precision | Grain texture, as-built shunt population |
| R3 Forgery asymmetry | Reproducing features in a physical artifact costs ≫ asset value | Requires controlling bulk semiconductor microstructure |
| R4 Challengeability | Measurement can be parameterized so static replicas fail | Bias-dependent current-path physics |
| R5 Evolution plausibility | Feature drift constrained by known physics, so histories are checkable | Crack/degradation physics is directional |
| R6 Field feasibility | Verification cost and time compatible with Section 1.3 economics | EL routine; magnetometric escalation (6.7) |

The table’s compression hides judgment calls that a reader adopting an alternative modality needs unpacked. R1’s “large” has a number behind it: the feature space must discriminate among the ~10⁹ units a global product line ships over its life, with FAR at the decision threshold small enough that expected collisions across the population are negligible — hundreds of effective bits, comfortably available in spatial defect structure and absolutely unavailable in scalar electrical parameters. R2’s “identifiable subset” is doing the honest work: no one claims the whole map is stable; the claim is that a decomposable subset is, and the requirement is really that the decomposition be defensible and versionable. R3 must hold against the strongest economically rational adversary of Chapter 8 — for modules that is A1/A2 with per-unit budgets in the tens of dollars, and the manufacture-a-matching-bulk-structure attack exceeds that by orders of magnitude; a modality whose forgery is a benchtop operation (surface texture, for an adversary who can re-laminate) fails R3 for exactly the high-value transactions that matter. R4 excludes modalities whose measurement admits a static simulacrum — a photograph fools a photograph. R5 is the requirement most proposals forget: a fingerprint whose legitimate drift is unmodeled cannot distinguish aging from substitution, and so fails precisely when the asset’s history becomes valuable. R6, finally, is the one requirement where the quantum modality currently scores worst, and the tier design of Section 6.7 is the architecture’s honest accounting of that fact rather than a workaround pretending otherwise.

## 6.6 The Verification Workflow: Trusting a History You Didn’t Witness

The point of all of it. A downstream party (D) — insurer, buyer, grid operator, adjudicator — holds none of the asset’s history and distrusts its custodian. The workflow by which (D) reaches justified confidence, at a chosen assurance level, decomposes trust into four separable questions, each answered by a different mechanism with a different failure mode: Is the record authentic? (cryptography and anchors). Is the record’s internal evidence sound? (policy over the graph). Does matter match record? (physical verification on a sound sample). What remains unknown, and what does it cost? (explicit residuals). The decomposition is the contribution; every step below is an implementation choice within it.

Step 1 — Record authenticity (no site visit). (D) resolves the asset DID, retrieves the event stream, checks signatures and corroboration classes, verifies digests against off-chain payloads, and verifies block inclusion against the public anchors (Section 4.4). Outcome: the history is exactly what was recorded, by the named parties, at the anchored times — regardless of what the consortium would now prefer. Operationally this is software, not diligence labor: the pilot insurer’s client (Section 11.3) runs Step 1 continuously across its whole book as a subscription service against headers and anchors, at a compute cost that rounds to nothing, so that by the time a human underwriter opens a file, authenticity is a solved input rather than a task.

Step 2 — Evidence-graph review (no site visit). (D) traverses the prior_refs DAG for the question at hand: does the commissioning event carry Class C corroboration? Were the instruments in calibration (their own DIDs, Step 1 applied recursively)? Do successive condition layers form a physically plausible evolution (R5)? Is the event cadence complete against the cohort baseline (Section 5.7)? Automated policy engines do this wholesale; Chapter 11 shows one. The step’s output is deliberately graded rather than binary — a policy score per asset with named deficiencies (retroactive registration class, an I-B instrument where policy prefers I-A, a cadence gap in years 7–9) — because (D)’s decision is a pricing decision, and pricing wants defects enumerated, not laundered into a pass/fail. The deficiencies also parameterize Step 3: a fleet whose graph review is clean draws the baseline sample; each named deficiency inflates the sample or targets it, so that physical verification spends itself where the paper is weakest.

Step 3 — Physical verification (site visit, sampled). For the assurance level the transaction warrants, (D) (or an accredited verifier (D) trusts) draws the challenge, measures the challenged slice in the field, and compares against the enrolled template chain. Sampling design — how many units, chosen how — is itself committed to the ledger before the visit, so the custodian cannot cherry-pick and (D) can later prove the rigor of its own diligence. Failures within the sample follow the escalation discipline of Section 3.6: re-measure (to exhaust instrument and registration error), escalate modality, then classify — honest mismatch with documented cause (an unlogged replacement, filed as a finding), or unexplained mismatch (a flag, a quarantine, and an inflated sample for the stratum, since one confirmed substitution converts the statistical question from “is the fleet clean?” to “how deep does this go?”). The verification campaign’s own events — design, measurements, findings — enter the assets’ records, so the next verifier inherits this one’s work.

Step 4 — Decision with quantified residuals. (D) now holds: authenticated history, policy-checked evidence graph, and physical binding verified on a random sample at stated false-accept rates. What remains unverifiable is enumerable (enrollment-time substitution; events off the ledger entirely) and priced rather than unknown.

### 6.6.1 The Sampling Arithmetic

Step 3’s economics rest on sampling statistics, and the arithmetic deserves one worked paragraph because it is where most of the cost savings live. Suppose (D) underwrites a 25,000-module plant and wants 95% confidence that fewer than 1% of units would fail physical verification (substitution, misidentification, or gross undisclosed damage). Under standard acceptance- sampling arithmetic, a random sample of roughly 300 units with zero failures delivers that assurance — 1.2% of the fleet, a few technician-days with a handheld V2 rig. Tightening to 0.5% at 99% confidence roughly triples the sample; loosening to lot-level questions (is this pallet as documented?) shrinks it to dozens. Stratification does real work: sampling within production weeks, installers, and site blocks converts the single fleet-wide question into cohort questions whose anomalies localize (a failure concentrates suspicion on its stratum, directing escalation). And the pre-commitment of the design — sample size, stratification, seed, drawn after the custodian’s records are frozen but before the site visit — is what converts these textbook numbers into adversarially valid ones: a custodian who cannot predict the sample cannot stage it, and a verifier who commits the design cannot be accused of cherry-picking failures afterward. The ledger holds both parties honest, which is the recurring pattern of the whole architecture in miniature. The design also travels well into legal settings: acceptance sampling against pre-committed designs is established practice in commercial quality disputes, so a Step-3 campaign arrives in arbitration wearing statistics the tribunal’s experts already know how to examine — one more instance of the book’s preference for grafting onto existing institutional competence over inventing new ceremony.

Figure 6.2 The four-step verification workflow. Steps 1–2 are remote and cheap; Step 3 is the proportional escalation of Section 3.6.

flowchart TB
    D[Downstream party D] --> S1[Step 1: Verify record authenticity<br>signatures, digests, public anchors]
    S1 --> S2[Step 2: Evidence-graph review<br>corroboration, calibration chains,<br>evolution plausibility, cadence]
    S2 --> DEC{Assurance level<br>required?}
    DEC -->|routine| S4a[Decide on remote<br>evidence alone]
    DEC -->|elevated| S3[Step 3: Sampled physical verification<br>pre-committed sampling design,<br>challenge-parameterized measurement]
    S3 --> S4[Step 4: Decision with<br>quantified residual risk]
    S4a --> OUT[Underwrite / price / accept / adjudicate]
    S4 --> OUT

What has actually changed relative to Chapter 1’s opening transaction deserves plain statement. The buyer’s engineer of Section 1.1 could only choose between trusting documents and re-testing everything. (D) instead verifies documents against mathematics (Steps 1–2, nearly free) and matter against documents on a random sample whose size is set by decision theory, not despair (Step 3). Trust has been decomposed into checkable parts, and the unverifiable residue has been made small, explicit, and insurable. That decomposition — not any single measurement — is the chapter’s contribution.

Run the Chapter 1 transaction’s numbers under the new regime to close the loop. The 10 MW acquisition’s status-quo diligence: a 200-module flash campaign at roughly USD 150 per unit all-in — USD 30,000 — answering the condition question weakly and the identity and completeness questions not at all, followed by a seven-figure residual-uncertainty discount. Under the workflow: Steps 1–2 run in software across all 25,000 modules for effectively nothing, answering identity-of-record and history-completeness questions outright; a pre-committed 300-unit V2 campaign at USD 10–20 per unit — USD 3,000–6,000 — answers the matter-matches-record question at 95/1 confidence; escalation reserves another few thousand for anomalies. Total verification spend falls by roughly 80% while the questions answered expand from one to three, and the residual discount compresses toward the genuinely unverifiable remainder (enrollment-era substitution, unrecorded events) — which Chapter 13 argues is precisely the fraction of the old discount an insurer will now underwrite for a fee, because for the first time it is enumerable. The seller of good assets recovers most of the information rent; the seller of bad ones finds the workflow pricing them accurately; and both outcomes are the market working, which is all this architecture ever promises.

## 6.7 Economics and Deployment Tiers

The mechanism must clear Section 1.3’s unit economics. The architecture’s answer is tiering — assurance purchased in proportion to value at risk, with each tier’s cost justified by the transaction it serves rather than spread across all units:

- Tier 0 (all units): enrollment at manufacture using in-line EL the factory already performs, plus flash-test VC. Marginal cost: template computation and a ledger event — cents. This tier alone, with no quantum instrument anywhere in the deployment, already defeats relabeling (the label no longer carries the identity), prices ghost-shift units (unregistered units are visibly unregistered), and gives every downstream transaction a structural reference — the majority of Section 1.4’s fraud economics, closed by the cheapest tier.

- Tier 1 (all units, opportunistic): re-verification whenever routine O&M imaging touches a unit anyway; the marginal cost of comparison against an existing image is near zero, and each match extends the asset’s documented-continuity thread (Section 6.4) at no cost — the compounding asset nobody budgets for and everybody later relies on.

- Tier 2 (sampled units, event-driven): magnetometric enrollment at manufacture for premium product lines, and field magnetometric verification for escalations — plant acquisitions, catastrophe claims, warranty adjudications — where the transaction at stake is many orders of magnitude above the measurement cost. A worked instance: a hail-catastrophe claim on a 100 MW plant turns on whether the crack population predates the storm; the insurer’s verifier pulls the pre-committed sample of sixty modules, runs V3 verification against enrollment and the year-6 condition records, and settles a seven-figure dispute on a five-figure measurement campaign — the Table 6.3 ratio in action.

- Tier 3 (reference): laboratory-grade characterization anchoring calibration chains and dispute resolution.

Instrument ownership across the tiers follows the interests: factories own Tier 0 (it is their QA line); O&M contractors own the Tier 1 imaging they already fly; Tier 2 instruments belong to verification service providers — the accredited, adversarially employable verifiers of Section 3.1.1 — because a verification performed with the custodian’s own instrument proves less by construction, and an independent verification industry is the institutional form the escalation tiers want (Chapter 13 notes it as a new market the architecture creates); Tier 3 lives with metrology institutes and the university labs of the pilot’s consortium. The accreditation machinery of Section 4.3.2 covers all of them.

The tier structure is why instrument maturity (Section 6.2’s honest statement) does not gate the architecture: Tiers 0–1 deploy on today’s mature EL infrastructure and already deliver most of the counterfeit and history-integrity value; Tier 2’s quantum layer raises the assurance ceiling for the transactions that pay for it, and widens as the instruments mature. Manufacturer adoption sequencing follows the same gradient: premium product lines enroll at Tier 2 first, because the magnetometric enrollment is a signal (Chapter 13’s vocabulary) — a verifiable claim of process quality that justifies premium pricing and that low-grade competitors cannot cheaply imitate — and the capability then diffuses down-market as instruments cheapen, the standard trajectory of quality instrumentation in this industry from EL itself onward.

Table 6.3 Tier economics, order-of-magnitude, per the deployment assumptions of Chapters 7 and 11. Costs in 2026 USD.

| Tier | Coverage | Marginal cost per unit | Cost bearer | Value protected per exercise |
| --- | --- | --- | --- | --- |
| 0 Enrollment | 100% at manufacture | < $0.10 (compute + ledger share) | Manufacturer (recovered in premium-line pricing) | Entire downstream evidence chain |
| 1 Opportunistic re-verify | Units touched by routine O&M imaging (~2–5%/yr) | ~$0 marginal (comparison against images already taken) | Owner/O&M (absorbed in existing campaigns) | Continuity of condition history |
| 2 Escalated verification | Sampled units at transactions/claims | $5–20 (V2 EL) to $50–300 (V3 magnetometric) | Transaction beneficiary | $10⁵–10⁸ per transaction |
| 3 Reference characterization | Handfuls of units; calibration and disputes | $10³–10⁴ per unit | Consortium / adjudication parties | Metrological root for every tier below |

Read the last two columns together and the design’s economic logic is complete: each tier’s cost lands on the party whose transaction consumes the assurance, at one to five orders of magnitude below the value it protects, and no tier’s cost is borne fleet-wide except the one (Tier 0) whose per-unit cost was engineered to near zero. This is the answer to Section 1.3’s unit-economics constraint, delivered not by making high assurance cheap — it is not — but by making sure nobody buys high assurance who does not need it that day.

## 6.8 Open Problems

Stated as research questions, continued in Chapter 14, each with its current state and the shape of a solution.

(1) Stability quantification. R2 needs twenty-year longitudinal data that does not yet exist. What exists: accelerated-aging studies (thermal cycling, damp heat, mechanical load sequences per the IEC 61215 family) through which enrolled templates have been tracked with encouraging grain-texture persistence; and the pilot’s 6,300-unit Tier-2 cohort, now accumulating real field years. What is missing: multi-climate, multi-technology cohorts old enough to expose slow mechanisms — and no acceleration protocol fully earns trust across twenty years, because acceleration factors are themselves models. The architectural hedge is supersession (Section 6.4); the scientific program is instrumented cohorts started now, which is the single highest- leverage investment available to this field.

(2) Inversion uniqueness. Field-to-current inversion is ill-posed at spatial frequencies above the standoff cutoff, and regularization fills the null space with assumptions. The security question — can an adversary exploit the null space to make one defect population impersonate another across the challenge space? — has not been formally posed, let alone bounded. The PUF community’s arc is the cautionary precedent: early PUFs claimed unclonability until machine-learning modeling attacks operationalized the question, and this modality should pre-empt the same arc by inviting the attack literature early. A solution looks like an adversarial bound: given measurement noise and challenge-space geometry, the minimum physical difference between populations distinguishable under the protocol.

(3) Throughput engineering. Bench wide-field systems image cells in minutes; a line-speed instrument needs two orders of magnitude, via larger NV-layer fields of view, parallel tiles, and optimized bias/readout sequences — engineering with identified paths and no new physics, awaiting an industrial customer to pay for it. The tier design means the architecture does not wait; but every factor of ten in throughput moves Tier 2 coverage from sampled toward universal and re-prices Table 6.3.

(4) Template standardization. Cross-vendor comparability of templates, similarity metrics, thresholds, and challenge protocols does not exist — every instrument vendor’s pipeline is currently its own island, which is tolerable in pilots and intolerable at market scale (Section 12.5 and standardization gap S3 develop the institutional argument). A solution looks like reference template formats and proficiency testing hosted by metrology institutes, in direct analogy to how dimensional metrology standardized.

(5) Thin-film and emerging cell technologies. The defect phenomenology of CdTe, CIGS, perovskites, and tandems is younger and different — different stochastic sources of individuality, different stability questions (perovskite ion migration is a moving substrate for any fingerprint), and different current-path geometries for inversion. Nothing in the architecture is c-Si-specific, but every calibration in this chapter is, and the honest statement is that each technology needs its own R1–R6 dossier before the binding claims transfer.

## 6.9 A Note on the Intellectual Property Posture

Readers evaluating this chapter for adoption deserve clarity about what the pending patent application does and does not mean for them, stated here once in plain language (the Preface carries the fuller context). The application covers specific mechanisms in the binding construction of Sections 6.4 and their embodiment in the verification workflow — the cross-modal locking with recorded co-registration, the challenge-parameterized measurement protocol, and the supersession-chain verification, as applied to photovoltaic devices. It does not cover, and could not cover, the general ideas this book situates them in: ledger-anchored asset identity, structural fingerprinting as a class, EL-based template matching, or the lifecycle schema — all of which have prior art this book cites and builds on openly. The requirements statement of Section 6.5 was written general precisely so that alternative mechanisms can be engineered against it without reference to the claimed construction, and Appendix B’s comparison table treats claimed and unclaimed mechanisms identically. My commercial position, for the record: the application exists to protect the possibility of a reference implementation being built to a standard rather than to enclose the field, and the standardization argument of Section 14.3 — that binding templates should be open metrology, not proprietary formats — is a position I hold against my own narrow interest where the two conflict. Readers may weigh the argument knowing its author’s stake; that is what the disclosure is for.

## 6.10 Chapter Summary

The defect structure of a photovoltaic device is manufacturing’s uncontrolled residue, and that is exactly what makes it the strongest available anchor for passive identity: unique by physics across grain structure, dislocations, precipitates, and process stochastics; unforgeable at reasonable cost by the R3 asymmetry, since reproducing a specified bulk defect population exceeds what process control achieves even for legitimate production; and — the design’s central economy — identical with the condition evidence the lifecycle record needs anyway. NV magnetometry supplies current-path maps that see beneath optical modalities, at room temperature and with a clear if unfinished road to line speed; the two-layer decomposition separates identity (stable structure) from condition (evolving damage), with the decomposition itself versioned as scientific understanding matures; cross-modal locking, challenge-parameterized measurement drawing on a bias-response surface never published in full, and supersession-with-provenance guarded by evolution-plausibility checks turn a measurement into a binding. On that binding stands the four-step verification workflow: authenticate the record against public anchors, police the evidence graph, verify matter against template on a pre-committed random sample, and decide with residual risk that is quantified rather than vague. Assurance is bought in tiers, so the mature EL layer carries routine load while the quantum-limited layer prices into the transactions that need it — with Tier 0 alone closing most of Chapter 1’s fraud economics before a single NV instrument ships. The chapter’s honest edges are marked: stability data that must be earned in calendar years (M1), an adversarial inversion question posed here before adversaries pose it (M2), throughput engineering awaiting an industrial customer (M3), a template-standardization gap with a closing window, and a technology dossier owed to every cell chemistry that is not crystalline silicon; the author’s stake is on the table in Section 6.9. What this mechanism defends, and what still gets through, is the business of Chapter 8 — after Chapter 7 confirms the whole design scales to fleets worth attacking.

## References and Further Reading

- Degen, C. L., F. Reinhard, and P. Cappellaro. “Quantum Sensing.” Reviews of Modern Physics 89, no. 3 (2017): 035002.

- Rondin, L., J.-P. Tetienne, T. Hingant, J.-F. Roch, P. Maletinsky, and V. Jacques. “Magnetometry with Nitrogen-Vacancy Defects in Diamond.” Reports on Progress in Physics 77, no. 5 (2014): 056503.

- Glenn, D. R., R. R. Fu, P. Kehayias, et al. “Micrometer-Scale Magnetic Imaging of Geological Samples Using a Quantum Diamond Microscope.” Geochemistry, Geophysics, Geosystems 18, no. 8 (2017): 3254–3267.

- Breitenstein, O., W. Warta, and M. C. Schubert. Lock-in Thermography: Basics and Use for Evaluating Electronic Devices and Materials. 3rd ed. Springer, 2018.

- Köntges, M., et al. Review of Failures of Photovoltaic Modules. IEA-PVPS Task 13 Report T13-01:2014 (crack morphology and evolution).

- Fuyuki, T., and A. Kitiyanan. “Photographic Diagnosis of Crystalline Silicon Solar Cells Utilizing Electroluminescence.” Applied Physics A 96 (2009): 189–196.

- Jain, A. K., A. Ross, and S. Prabhakar. “An Introduction to Biometric Recognition.” IEEE Transactions on Circuits and Systems for Video Technology 14, no. 1 (2004): 4–20 (false-accept/false-reject formalism).

- Schilling, D. R., and D. V. Neubauer. Acceptance Sampling in Quality Control. 3rd ed. CRC Press, 2017. The sampling-design basis of Section 6.6.1.

- International Electrotechnical Commission. IEC 61215 series (design qualification test sequences — the accelerated-aging protocols behind the M1 stability evidence). Geneva: IEC.

- [AUTHOR LAST NAME], [FIRST NAME]. [PATENT APPLICATION TITLE — TO BE SUPPLIED]. Application No. [NUMBER — TO BE SUPPLIED], filed [DATE — TO BE SUPPLIED]. Covers binding and verification workflow in Sections 6.4–6.6.