# Blockchain-Based Identity and Provenance for Distributed Energy Assets

Immutable Identity, Lifecycle Traceability, and Integrity Verification for Solar and Renewable Infrastructure

[AUTHOR NAME — TO BE SUPPLIED]

Table of Contents

# Physical Asset Identity Models

## What This Chapter Covers

Chapter 1 defined what an asset identity must provide — uniqueness, persistence, binding, attributability — and Chapter 2 supplied the record-keeping machinery. This chapter addresses the piece the machinery cannot supply: how a digital identity is attached to a physical object at all. It develops three bodies of work in turn — digital twins, cryptographic device identity rooted in hardware, and the W3C identity standards (decentralized identifiers and verifiable credentials) — and adapts each from its native context to energy hardware. The chapter’s central distinction, between active binding (the device holds a key and can prove it) and passive binding (the device’s measurable structure serves as its credential), organizes everything that follows and sets up Chapter 6. Along the way it fixes the statistical and adversarial refinements of the binding problem (Section 3.1.1), draws the twin/spine boundary that keeps operational and evidential data in their proper places (Section 3.2), walks the active-binding lifecycle that the meter fleets have already proven at national scale (Section 3.3), formalizes the passive template pipeline (Section 3.4), adapts the W3C naming and claims standards to subjects that are objects (Section 3.5), and closes with the composition rules — the V0–V3 escalation ladder and the aggregate-identity test — that turn mechanisms into deployments (Section 3.6).

## 3.1 The Binding Problem Stated Precisely

Suppose the consortium ledger of Chapter 2 exists and functions perfectly. A record says: asset A-3F92… was flash-tested at 402.1 Wp on 14 March 2026, signed by manufacturer M. A field engineer stands in front of a module. The ledger cannot answer the engineer’s actual question: is this laminate the asset the record designates?

Formally, binding requires a verification function (V(, ) {, }) with two properties:

- Soundness. A substituted or counterfeit object yields no match except with negligible probability — and, critically, the cost of manufacturing an object that yields a false match exceeds the value of doing so.

- Field feasibility. (V) is executable where the asset lives — a rooftop, a desert plant, a warehouse — at a cost per verification compatible with Section 1.3’s unit economics, by a party who need not trust the asset’s custodian.

Every identity technology in this chapter is an implementation of (V), and every one occupies a different point on the trade-off surface spanned by soundness, cost, and the demands placed on the object itself. The taxonomy in Figure 3.1 previews the chapter. It is worth pausing on how unusual the question is: essentially all of applied cryptography assumes the endpoint is the keyholder — authenticate the key and you have authenticated the party. Physical-asset identity breaks that assumption at the root, because the entity of interest holds no keys, computes nothing, and cannot participate in its own authentication. Everything distinctive in this chapter flows from taking that breakage seriously instead of papering over it with a label that holds the keys on the object’s behalf — which is, in one sentence, the design error of every artifact-based scheme in the left branch of the taxonomy.

Figure 3.1 A taxonomy of binding mechanisms for physical assets. The right-hand branch — passive structural fingerprints — is where Chapter 6’s contribution sits.

flowchart TB
    ROOT[Binding mechanisms] --> ART[Artifact-based:<br>labels, tags, seals]
    ROOT --> ACT[Active cryptographic:<br>device holds a secret]
    ROOT --> PAS[Passive structural:<br>device IS the credential]
    ART --> ART1[Barcode / QR serials]
    ART --> ART2[RFID / NFC tags]
    ART --> ART3[Tamper-evident seals]
    ACT --> ACT1[Secure element /<br>TPM device keys]
    ACT --> ACT2[PUF-derived keys]
    PAS --> PAS1[Optical fingerprints:<br>EL crack patterns,<br>surface texture]
    PAS --> PAS2[Electrical signatures:<br>dark I–V, capacitance]
    PAS --> PAS3[Quantum-sensed<br>defect maps — Ch. 6]
    style ART fill:#f3d9d3,stroke:#9c4a3c
    style ACT fill:#dbe9f6,stroke:#2c5f8a
    style PAS fill:#d9ead9,stroke:#3c7a3c

Artifact-based mechanisms were dispatched in Section 1.5 — they are bearer tokens, copyable by construction — and receive no further systematic treatment, though Section 3.6 notes their legitimate residual role as cheap locators (a QR code that tells the verifier which identity to check is useful even though it proves nothing).

### 3.1.1 Refining the Requirements: Errors, Adversaries, and Who Holds the Instrument

The two-property statement of (V) above is the executive version; engineering needs three refinements before the mechanisms can be compared honestly.

First, (V) is statistical, not Boolean. Every physical measurement carries noise, every physical object drifts with age, and so every real verification function has a false-accept rate (FAR: a wrong object passes) and a false-reject rate (FRR: the right object fails), traded against each other by a decision threshold. The two error types have asymmetric costs that depend on the use: at a warranty adjudication, a false accept pays a fraudulent claim, while a false reject wrongly denies an honest one; at a routine custody scan, a false reject merely triggers a second measurement. Mechanism comparisons that quote a single “accuracy” number are hiding this trade-off, and the biometric community’s receiver-operating-characteristic discipline — publish the curve, not a point — is the standard this book applies to its own proposals in Chapter 6.

Second, soundness must be stated against an adversary model, not against chance. The probability that two honest modules collide in fingerprint space (random collision) is a different quantity from the probability that a motivated forger, holding the published template and a budget, produces an object that passes (adversarial collision) — and it is the second quantity that secures anything. The gap between the two is where artifact mechanisms die: barcode collisions are vanishingly rare by chance and universal under adversaries. Every soundness claim in this chapter and Chapter 6 is an adversarial claim, priced in attacker cost, and Chapter 8 supplies the attacker taxonomy those prices assume.

Third, the verifier’s instrument is part of the trusted base. A verification performed with the custodian’s own camera, on the custodian’s premises, under the custodian’s lighting, proves less than the same measurement with the verifier’s sealed instrument — and the difference is not paranoia but the scene-spoofing attack family of Chapter 8. This is why the oracle rules of Chapter 4 push signing into instruments, why instruments themselves carry identities and calibration lifecycles, and why the field-feasibility requirement includes “by a party who need not trust the asset’s custodian.” A binding mechanism whose verification cannot be performed adversarially is a demonstration, not a control.

With those refinements, the comparison surface for everything that follows has five axes: FAR/FRR behavior under field conditions; adversarial forgery cost; per-unit enrollment cost; per-verification cost and time; and longevity of the underlying feature. Appendix B tabulates all the mechanisms of this chapter and Chapter 6 against exactly these axes, and readers who want the scorecard before the argument may detour there now.

## 3.2 Digital Twins, and What Identity Adds to Them

The digital twin literature, originating in aerospace lifecycle management, envisions a virtual counterpart of a physical asset, updated by data flowing from the asset and used for simulation, prediction, and decision support. Much of the DER industry already operates something twin-like: inverter vendors maintain per-device telemetry histories; plant asset-management platforms hold component registers with maintenance records.

The lineage is worth one sentence of respect before the critique: the twin concept earned its keep in programs where a physical article’s virtual counterpart carried real engineering authority — configuration-managed, its divergence from the article a reportable event — and that discipline, more than the technology, is what the DER industry’s twin-like platforms have not inherited. It is tempting to say the asset identity record of this book is a digital twin, but the equation obscures the two properties twins conventionally lack. First, twins are operational artifacts, owned and mutable by whoever operates the platform — they optimize performance, not evidence. Nothing prevents retroactive revision, and their lifetime is the platform subscription. Second, twins inherit their association with the physical asset from the same fragile channels (serial numbers, commissioning spreadsheets) whose failure Chapter 1 documented; a twin faithfully mirrors whatever device its telemetry stream happens to come from.

The synthesis this book adopts: the ledger-anchored identity record is the evidential spine — sparse, append-only, multi-party, bound to the physical unit — while operational twins remain where they are, in vendor and operator platforms, but hang their data off the spine by reference and digest (the partitioning mechanics are Chapter 4’s subject). The twin answers “how is the asset performing?”; the spine answers “which asset is this, and who said what about it, when?” Conflating the two questions produces systems that answer neither reliably.

The failure modes of conflation run in both directions and are worth a paragraph each, because both are being built commercially as this book is written. Making the twin evidential — pushing telemetry streams onto a ledger — inherits every cost of consensus replication for data that needs none of it: nobody adjudicates a warranty on a fifteen-minute power reading, and Chapter 4’s workload table will show the volumes are prohibitive by five orders of magnitude anyway. Worse, it creates a false sense of evidence: an immutable record of telemetry from an unbound device is Chapter 1’s garbage-in permanence problem with a subscription fee. Making the spine operational — treating the identity record as the live asset-management database — fails the other way: operational data models change monthly with business needs, and a record layer that must be governed by consortium vote cannot iterate at that pace, so either governance collapses into a rubber stamp (losing the multi-party guarantee) or the operators route around the system (losing the data). The boundary that works, and that Chapter 5’s event schema formalizes, is decision-grade summaries cross the line; everything else stays operational: the commissioning test result enters the spine, the per-string debug data behind it stays in the twin, and a digest-plus-reference connects them for the auditor who someday wants both.

Interoperability standards for twins deserve a note, because the two communities are converging on this book’s territory from the other side. The industrial digital-twin bodies — the asset administration shell (AAS) work in the Industrie 4.0 ecosystem being the most developed — define machine-readable models of asset properties and submodels, and the EU product-passport regulations increasingly gesture at them as candidate data models. Nothing in that work conflicts with this book; what it lacks, uniformly, is the evidential layer — binding, non-suppression, consensus ordering, institutional survivability — and what this book lacks, deliberately, is their rich operational semantics. The projection mapping of Chapter 10 (ledger events onto passport fields) is exactly the seam where the two meet, and Chapter 14 lists the standardization work the seam still needs.

Table 3.1 Operational digital twin versus evidential identity record.

| Dimension | Operational twin | Evidential identity record |
| --- | --- | --- |
| Primary purpose | Performance, prediction, O&M | Evidence: identity, provenance, condition history |
| Update rate | Continuous telemetry | Sparse lifecycle events |
| Mutability | Mutable, single-operator | Append-only, multi-party |
| Lifetime | Platform / contract term | Asset lifetime (decades) |
| Physical binding | Inherited from serials | Explicit mechanism (this chapter, Ch. 6) |
| Typical owner | Vendor or operator | No single owner (consortium) |

## 3.3 Active Binding: Device Keys and Hardware Roots of Trust

For assets with electronics and power, the mature answer is a device identity key: an asymmetric key pair generated inside, and never leaving, a tamper-resistant hardware element. The public key (or a certificate over it) becomes the asset’s identifier; the verification function (V) is a challenge–response — the verifier sends a random nonce, the device signs it, the signature checks against the registered public key. Soundness reduces to the difficulty of extracting the private key from the silicon, which dedicated secure elements make genuinely expensive: protected key storage, side-channel countermeasures, and attestation of the firmware the device booted.

Three grades of this idea appear in energy hardware, in descending order of assurance: discrete secure elements or TPMs (the strongest, standard in recent smart meters under schemes such as the German BSI metering regime); key material in the main processor’s trusted execution environment (weaker isolation, no extra part cost); and soft keys in ordinary flash (little more than an obfuscated serial number — extraction is a firmware bug away). A fourth approach generates the key from a physically unclonable function (PUF) — manufacturing variation in silicon read as a device-unique value at each boot, so no key is stored at rest. PUFs are attractive on paper for cost-sensitive devices, with the caveats that raw PUF responses are noisy (requiring error-correcting “helper data” whose own integrity must be managed) and that aging drift over multi-decade service lives remains an active research concern — a caveat that recurs, in a different guise, for the passive fingerprints of Section 3.4.

### 3.3.1 What the Meter Fleets Already Proved

Active binding at national scale is not a proposal; it is an operating fact, and the smart-metering programs are its proof. The German metering regime is the strictest instance: the BSI’s protection profiles require certified security modules in every smart meter gateway, keys provisioned under audited ceremonies, and cryptographic identity woven into the market’s data flows — and the fleet runs at millions of units under exactly the multi-decade, multi-custodian, regulatorily supervised conditions this book cares about. Comparable if less prescriptive regimes operate across the UK (where the central data communications company terminates device trust), parts of Scandinavia, and the advanced state programs in the US. Three transferable lessons emerge from that operating history. First, unit cost collapses with scale and mandate: security silicon that pilot programs priced in dollars settled to well under one dollar at fleet volumes, and the certification overhead — the real cost — amortized across vendors once profiles stabilized. Second, the hard operational problem is key ceremony logistics, not cryptography: provisioning stations, personnel vetting, and the handling of failed provisioning runs generated most of the field incidents, which is why Section 4.3 treats issuance as the trusted setup and why the pilot of Chapter 11 staffs it accordingly. Third, governance outlives hardware: meter fleets have already crossed vendor bankruptcies, protocol revisions, and one full algorithm generation, and the programs that handled these smoothly were those whose device identities were registered against the program’s infrastructure rather than any vendor’s — the institutional-independence argument of Chapter 1, validated in production. The DER asset classes with electronics can, in effect, inherit a proven playbook; this book’s contribution for them is mainly to connect that playbook to the evidential record layer. The module cannot inherit it, which is why the passive sections of this chapter and Chapter 6 exist.

The applicability boundary is sharp and shapes the whole architecture:

Table 3.2 Active binding applicability across the DER asset population.

| Asset class | Electronics / power | Active binding | Notes |
| --- | --- | --- | --- |
| Inverters (string, central, micro) | Yes | Well suited | Secure element adds ~USD 1–3 per unit |
| Battery management systems | Yes | Well suited | Key per pack; cell-level binding still open (Ch. 12) |
| Smart meters, gateways, controllers | Yes | Established practice | Regulatory precedent exists |
| Trackers, combiner monitoring | Usually | Feasible | Retrofit fleet is the obstacle |
| PV modules | No | Not applicable | Passive laminate; the central hard case |
| Mounting, cabling, structural BOS | No | Not applicable | Low unit value; batch identity usually suffices |

Because active binding carries so much of the architecture for the electronic asset classes, its operational lifecycle deserves more than the summary above; the following paragraphs walk it end to end, and the reader will recognize each stage as the active-device mirror of what Chapters 4–6 build for passive ones.

Provisioning. The key pair is generated inside the secure element on the production line — never injected from outside, since an injected key existed somewhere else at least once — and the public half is captured into the device’s registration along with the element’s own attestation certificate, which chains to the silicon vendor. Provisioning is the active device’s enrollment moment and shares the trusted-setup character of Section 4.3: subvert the line here and every later verification faithfully authenticates the wrong association. The mitigation is the same recursion used everywhere in this book — the provisioning station is itself an identified, attested instrument.

Attestation in operation. A live device can prove more than possession of its key: measured boot and remote attestation let it sign a statement of what firmware it is running, which grid operators increasingly have independent reasons to demand (Chapter 10’s interconnection interfaces; Chapter 12’s firmware-churn problem). The commissioning self-tests of Chapter 11 lean on this: the inverter’s commissioning event is signed by the device, under firmware whose hash is in the event, through a key that never left the silicon — a chain with no human transcription anywhere in it.

Key lifecycle over decades. Certificates expire, algorithms age (the ECC roots in today’s elements are Chapter 9’s most exposed row), elements fail, and boards get replaced in field service. Each has an answer with a schema hook: expiry and renewal are accreditation-register events; algorithm migration rides the hardware-refresh re-attestation pattern (Section 9.4, P3); and board replacement is the interesting one — the device identity does not survive replacement of the element that holds its key, so the service event records a successor identity linked to the predecessor, exactly the supersession-with-provenance pattern that passive re-enrollment uses. The symmetry is not cosmetic; it lets one verification logic serve both binding families.

The largest asset population by count — the module — falls outside the active paradigm. Proposals to embed powered electronics in module junction boxes exist (module-level power electronics already put silicon there), but they raise cost, add a failure mode to a 30-year passive device, and protect only units built henceforth, doing nothing for the installed terawatt. A per-module secure tag at even one dollar is a seven-figure line item on a large plant for which the buyer receives protection of the tag, not the laminate — the tag-to-laminate association being exactly as forgeable as the label it replaced unless the tag’s placement is itself tamper-evident, which returns the problem to physical binding by another road. The passive route is not a fallback; for modules it is the main road.

## 3.4 Passive Binding: The Object as Its Own Credential

A passive binding mechanism derives the asset’s credential from measurable physical structure — no electronics, no stored secret. The general pattern has three parts:

- Enrollment. At a trusted moment (manufacture, or first inspection), measure a structural property of the unit; reduce the measurement to a fingerprint template; commit the template (or its digest) to the identity record.

- Verification. In the field, re-measure; compare against the enrolled template under a similarity metric with a decision threshold.

- Robustness engineering. The hard part: the fingerprint must be stable under legitimate aging and measurement noise, yet discriminating against other units and expensive to forge — the same triangle biometrics has negotiated for decades, and the false-accept/false-reject formalism of that field transfers directly.

Candidate fingerprints for PV modules, roughly in order of current practicality:

- Electroluminescence (EL) structure. Under forward bias in darkness, a module’s cells emit near-infrared light whose spatial pattern reveals crystal grain structure, micro-cracks, and finger defects — patterns fixed at manufacture and effectively unique per cell. EL imaging is already routine in factory QA and field inspection, so the instrument base exists. The obstacle is stability: cracks evolve (transport, hail, thermal cycling), so naive template matching degrades precisely when the asset’s history gets interesting. A workable scheme must separate stable structural features (grain boundaries, as-built crack morphology) from evolving damage — and, done well, this turns the liability into the product: the difference between enrollment and verification images is itself the degradation evidence the lifecycle record wants (Chapter 6 builds exactly this).

- Electrical signatures. Dark I–V curves, series/shunt resistance profiles, and capacitance spectra are cheap to measure at the string or module level but are low-dimensional — discriminating among thousands of nominally identical units is marginal — and drift with the very degradation the system must tolerate. Useful as corroboration, insufficient alone.

- Surface and material texture. Optical speckle from encapsulant or backsheet texture, in the spirit of “fingerprints of paper” work in physical cryptography; high entropy, but read points must be relocatable after decades of weathering, and outdoor soiling is unforgiving.

- Structural defect maps. Magnetometry with nitrogen-vacancy centers and related quantum-limited modalities map current-flow anomalies and material structure at resolution and depth unavailable to optical methods alone; classical EL and thermography provide corroboration at factory and field scale (Chapter 6, Section 6.0). The maps capture features intrinsic to the semiconductor bulk — the hardest layer of the device for a forger to reproduce. This is the anchor of the patent-pending approach this book develops; Chapter 6 treats it in full, including the sober accounting of instrument cost and throughput that any honest proposal owes.

The four candidates deserve a closer physical look than the bullet summaries, because their strengths are set by where in the device their features live. EL structure is a junction phenomenon: the emission intensity at each point tracks local radiative recombination, so the image is a map of electronic quality across the cell — grain boundaries appear because recombination differs across crystal orientations, cracks appear where the emission path is broken, and process signatures (firing profiles, passivation variation) leave batch- level texture on which unit-level individuality rides. Its blind spots follow from the same physics: features that do not modulate radiative recombination — a resistive solder bond early in its life, a buried shunt below the junction — are dim or invisible. Electrical signatures integrate over the whole device: every defect contributes to the dark I–V curve, but summed into a handful of scalar parameters, which is why their entropy is low — a million modules project onto a few distinguishable curve families. Surface texture lives at the outermost layers, which is what makes it both cheap to read and cheap to attack: it is the only candidate an adversary can replace (re-laminate a backsheet, polish and re-texture) without touching the functional device. Defect maps read by magnetometry live deepest — in the current paths through the semiconductor bulk and metallization — which is the root of both their forgery resistance and their instrumentation cost. The pattern to notice: depth buys security and costs accessibility, and the composite designs of Section 3.6 and Chapter 6 are ways of buying depth only when a transaction justifies it.

The enrollment moment deserves separate attention for passive mechanisms, because it is where their guarantees are weakest and their economics are decided. Enrollment at manufacture — in-line, as part of the QA imaging the factory already performs — costs nearly nothing, captures the device in its best-documented state, and inherits the factory’s controlled measurement conditions; it is the gold standard, and Section 4.3 builds the issuance protocol around it. Enrollment in the field — the retrofit case, unavoidable for the installed terawatt — costs a site visit, captures a device whose prior history is unknowable, and must be honest about that: the resulting identity attests continuity from enrollment forward, nothing earlier. The two classes must be distinguishable in the record forever (Chapter 4’s registration classes), and the market must price them differently (Chapter 13 confirms it will). What retrofit enrollment buys, despite the caveats, is substantial: it stops the clock on further record loss, enables every downstream verification the factory-enrolled fleet gets, and — where the retrofit measurement includes condition imaging — converts the unknown prior history into a quantified present condition, which is most of what a buyer or insurer actually needs.

### 3.4.1 The Template Pipeline, Formally

The three-part pattern above hides a pipeline whose stages each carry design decisions; naming them here saves Chapter 6 from re-deriving them. Acquisition produces a raw measurement (m) under recorded conditions (c) (bias, temperature, geometry — recorded because comparison across conditions is a leading error source). Registration maps (m) onto the object’s intrinsic coordinate frame — for a module, the cell grid and busbar skeleton — so that templates compare feature-to-feature rather than pixel-to-pixel; registration failures masquerade as identity failures, and robust fiducials are worth more than sensor resolution. Feature extraction reduces the registered measurement to a vector (t) designed for three properties in tension: high entropy across units (discrimination), low variance within a unit over time and instruments (stability), and physical interpretability (so that Chapter 6’s evolution- plausibility checks have meaning). Comparison computes a similarity (s(t, t’)) against the enrolled template and applies the threshold () whose FAR/FRR trade Section 3.1.1 introduced — with () set per use case, not per mechanism: the same fingerprint system legitimately runs a loose threshold at a warehouse gate and a tight one at an adjudication.

Two quantities summarize a mechanism’s quality and appear in Appendix B. The effective entropy of the template — how many bits of device-individuality survive extraction — bounds discrimination: a dark I–V trace yields a handful of stable bits (hence “insufficient alone” above); an EL grain map yields hundreds. And the stability margin — the gap between within-unit drift and between-unit distance over the service life — is the quantity nobody yet has twenty-year data for, which Chapter 6 flags as open problem M1 and hedges with supersession. The formalism, incidentally, is the PUF literature’s, transplanted: a passive fingerprint is best understood as a non-electronic PUF read by an external instrument, and that community’s error-correction and helper-data machinery — along with its sobering history of modeling attacks — transfers with the name.

The forgery economics deserve emphasis because they differ fundamentally from active binding. Extracting a key from a secure element is an attack on one chip, and a success is silent. Forging a structural fingerprint means manufacturing a semiconductor device whose internal defect distribution matches a published template — an act that current process control cannot achieve even for the legitimate manufacturer, since the defect distribution is precisely what production does not control. Section 8.3 formalizes this asymmetry; it is the strongest single argument for structural binding of high-value passive assets.

A precedent from outside the sector calibrates confidence. The “fingerprints of paper” line of work showed two decades ago that ordinary paper’s fiber structure, read by commodity optics, yields fingerprints with enormous entropy, robustness to handling and soiling, and no two sheets alike — and that the result generalized to plastics and packaging. Nobody has demonstrated economical forgery of such structural fingerprints since, across twenty years of anti-counterfeiting deployment. Semiconducting laminates are a harder measurement environment than paper but a better substrate for the argument: their structure is three-dimensional, electrically functional, and bound into the object’s performance, so the forger must reproduce not merely an appearance but a behavior. The reader should hold the analogy loosely — Chapter 6 does the domain-specific work — but the direction of the evidence is uniform: structural individuality is cheap to read and expensive to fake, which is exactly the asymmetry an identity system wants.

## 3.5 Names and Claims: DIDs and Verifiable Credentials for Hardware

Binding attaches a record to an object; something must still say what the identifier looks like and how statements about it are expressed so that parties who share nothing but standards can interoperate. The W3C’s decentralized identifier (DID) and verifiable credential (VC) specifications are the serious candidates, designed originally around persons and organizations. Applying them to hardware is mostly straightforward and instructive where it is not.

A DID is a URI (e.g., did:method:identifier) resolvable — via a method specification — to a DID document listing public keys and service endpoints. For an asset, the natural design registers a DID per unit, with the DID document holding the active-binding public key (where one exists), pointers to the passive-fingerprint enrollment digest, and the ledger address of the lifecycle record. The DID method would resolve against the consortium ledger, making the ledger the registry of Chapter 2’s registrar role.

Where the person-centric assumptions creak, the failures are informative:

- No self-sovereignty. The ideology of DIDs is that the subject controls the identifier. A laminate controls nothing. The controller of an asset DID is necessarily some party — and it must change at every custody transfer of Figure 1.1. Controller succession, an afterthought in person-centric DID methods, becomes a first-class lifecycle event (Chapter 5 gives it a schema).

- Key rotation without a subject. A person whose key is compromised rotates it. A module’s “key” is its structure; passive fingerprints are re-enrolled (a new measurement supersedes, with provenance, the old) rather than rotated, and the DID document must represent both live and superseded bindings with their intervals of validity — which is also exactly the hook Chapter 9 needs for algorithm migration.

- Resolution over decades. A DID is only as persistent as its method’s infrastructure. This re-raises Section 1.3’s institutional-lifetime problem one level up, and is among the standardization gaps Chapter 14 lists.

The resolution problem deserves the extra paragraph, because it is the one that person-centric deployments have quietly failed to solve and that a thirty-year asset registry cannot defer. A DID’s promise is that did:method:identifier resolves — today, and in 2050 — to an authoritative document. But resolution runs through the method’s infrastructure: a ledger that must still have readable replicas, a specification that must still have conforming implementations, a community that must still exist. Several early DID methods are already effectively unresolvable, their ledgers abandoned — harmless for the experimental credentials they carried, instructive for us. The design consequence adopted here is resolution independence of verification: every verification workflow in this book (Chapter 6’s Steps 1–4, Appendix A’s verifier) is constructed so that a verifier holding the asset’s record bundle — events, payloads, proofs, anchors — can verify everything without live resolution, treating the DID as a name within the bundle rather than a service to call. Live resolution is a convenience of the healthy years; the evidence must not depend on it. The bundle format that makes records portable across resolver death is one more item on Chapter 14’s standardization list (S2), and its absence from current standards is among this book’s specific criticisms of them.

A concrete sketch fixes ideas. An asset DID document, in the consortium method this book assumes, resolves to something of this shape (illustrative, not normative — the standardized form is Chapter 14’s item S2):

{
  "id": "did:derc:asset:a3f92c...",
  "controller": "did:derc:role:owner-r4471",
  "controllerHistory": [
    { "controller": "did:derc:role:mfr-m0021",
      "from": "2027-03-14", "until": "2027-09-02",
      "successionEvent": "evt:7f31..." },
    { "controller": "did:derc:role:owner-r4471",
      "from": "2027-09-02", "successionEvent": "evt:9a04..." }
  ],
  "binding": [
    { "type": "StructuralTemplate/EL-v2",
      "templateDigest": "b1946ac9...",
      "enrolled": "2027-03-14", "status": "active",
      "supersedes": null },
    { "type": "StructuralTemplate/QDM-v1",
      "templateDigest": "5d41402a...",
      "enrolled": "2027-03-14", "status": "active",
      "coRegistration": "c0ffee11..." }
  ],
  "lifecycleRecord": "ledger:derc-main/asset/a3f92c...",
  "algorithmPolicy": "policy:2027-baseline"
}

Every field earns its place in a later chapter: controllerHistory is the succession machinery Chapter 5 events feed; the binding array holds live and superseded templates with the co-registration reference that Chapter 6’s cross-modal locking requires; algorithmPolicy is Chapter 9’s migration hook. The reader should notice what is absent — no owner names, no site address, no commercial terms — which is Chapter 10’s privacy design showing through the naming layer.

A verifiable credential is a signed, schema-conformant set of claims by an issuer about a subject — precisely the right container for the sector’s existing paper: flash-test certificates (issuer: manufacturer), commissioning reports (issuer: EPC), inspection findings (issuer: independent engineer), warranty registrations. VCs give these documents cryptographic authorship and integrity, selective disclosure (Chapter 10 leans on this), and machine-checkable schemas — while the ledger supplies what VCs alone lack: ordering, timestamping against consensus time, non-suppression (a revoked-then-hidden credential is visible as a gap), and a registry to bind the credential’s subject to a physical unit. The complementarity is exact, and Figure 3.2 assembles the pieces into the reference stack the rest of the book assumes.

### 3.5.1 A Credential’s Life, Walked Through

Because verifiable credentials will carry most of the sector’s existing paperwork in Part II, one credential’s full lifecycle is worth walking end to end — the flash-test certificate, the sector’s most-forged document.

Issuance. The flash tester (an instrument with its own DID and calibration history) measures the module; the manufacturer’s issuing service constructs a credential whose subject is the module’s DID, whose claims are the measured parameters under standard test conditions, and whose issuer field is the manufacturer’s role DID; the credential is signed under the algorithm policy of the day. Critically, the credential also references the instrument’s attestation of the raw measurement — so the human-institutional claim (“we certify 402.1 W”) and the machine claim (“I measured 402.1 W”) are separately attributable, and a dispute can distinguish instrument error from institutional fraud.

Anchoring. The credential’s digest rides the registration event onto the ledger. The credential itself is a payload — off-chain, access-controlled — but its existence, timing, and issuer are now facts no one can quietly amend. A forger can still fabricate a plausible-looking certificate; what the forger cannot do is retro-insert its digest into an anchored history, which means every forged certificate is detectably absent from the record it claims membership in. This absence-detection is the ledger’s whole contribution to the VC layer, and it is decisive: document fraud in the sector has always relied on the impossibility of checking a document against a complete, trustworthy index.

Presentation. Years later, the owner presents the credential to an insurer — selectively: the power bin and measurement date, say, without the full I–V trace the manufacturer considers proprietary. The VC data model’s selective-disclosure mechanisms make the redacted presentation verifiable against the original signature, and Chapter 10 leans on exactly this to reconcile evidence with confidentiality.

Revocation and correction. The manufacturer discovers a miscalibrated tester and must retract a week of certificates. Revocation is itself an event — a signed, anchored statement referencing the affected credentials and the reason — and the corrected reissues reference both the originals and the revocation. No history is erased; the record shows a measurement, a discovered fault, and a correction, each attributed and timed, which is precisely what an evidential system should show. Contrast the status quo, where the same correction is an email asking recipients to discard the old PDF.

Issuer death. The manufacturer eventually dissolves. Its issuer key expires with it, but every credential it ever issued remains verifiable — signature against the algorithm policy of its era, existence against the anchored history, authority against the accreditation register showing the issuer’s status at issuance time. The credential has outlived the institution, which is Section 1.3’s requirement, met.

The same lifecycle serves the sector’s other paper with the nouns changed: commissioning reports (issuer: EPC; co-issuer: owner’s engineer), inspection findings (issuer: independent engineer; instrument attestations attached), warranty registrations (issuer: manufacturer; subject: module-and-site pair), recycler declarations (issuer: accredited recycler; mass-balance claims). What the pattern replaces, in every case, is a PDF whose authority was its letterhead.

One dependency should be named before it bites in Part II: credentials are only as interoperable as their claim vocabularies. A flash-test VC is useful across organizational boundaries only if “maximum power at STC” means the same field, unit, and measurement procedure to every consumer — which is a standards problem, not a cryptography problem. The sector is better positioned here than most: IEC test procedures already define the semantics of nearly every claim the credentials need to carry (61215 qualification results, 60904 measurement methods, 62446 commissioning tests), so the vocabulary work is largely transcription of existing normative documents into schema form rather than invention. Where the transcription has not happened — degradation-rate claims, battery state-of-health, structural-template metadata — every consortium currently invents its own fields, and Chapter 14 lists the harmonization as standardization gap S1’s payload half. The architecture tolerates the interim untidiness by versioning payload schemas per event type (Chapter 5), but tolerance is not endorsement, and procurement teams evaluating systems in this space should ask to see the schema registry before the consensus benchmark.

Figure 3.2 The layered identity stack assumed throughout Parts II–IV. Standards supply the naming and claim formats; the ledger supplies ordering and permanence; the binding layer attaches the whole construction to matter.

flowchart TB
    subgraph L4 [Claims layer]
        VC1[Verifiable credentials:<br>test reports, commissioning,<br>inspections, warranty]
    end
    subgraph L3 [Naming layer]
        DID[Asset DIDs + DID documents:<br>controller, binding references,<br>succession history]
    end
    subgraph L2 [Record layer — Ch. 2, 4, 5]
        LED[Consortium ledger:<br>registry, lifecycle state machine,<br>anchored to public chain]
    end
    subgraph L1 [Binding layer — this chapter, Ch. 6]
        AB[Active: device keys<br>in secure elements]
        PB[Passive: structural<br>fingerprints / defect maps]
    end
    VC1 --> DID
    DID --> LED
    LED --> AB
    LED --> PB
    AB --> HW[Physical asset]
    PB --> HW

## 3.6 Composite Identity in Practice

Real deployments compose mechanisms, because the mechanisms fail differently. The composition rule this book adopts — developed operationally in Chapters 4 and 11 — is: locate cheaply, verify proportionally. A printed QR code locates the identity record in seconds and proves nothing. Routine custody transitions verify against the cheap layer appropriate to the asset class (challenge–response for an inverter; a handheld EL spot-check against enrolled structure for a sampled subset of modules). High-stakes transitions — warranty adjudication, plant acquisition, insurance underwriting after a catastrophe claim — escalate to full structural verification of statistically chosen samples, with the sampling design itself recorded on the ledger so that the verification’s rigor is later provable. Identity assurance, like every other engineering quantity in this book, is purchased in proportion to the value at risk.

Table 3.3 makes the escalation ladder explicit for the module case; the tier labels (V0–V3) recur in Chapters 6, 7, and 11, where the enrollment side gets a matching ladder.

Table 3.3 Verification escalation ladder for PV modules under composite identity. Costs are order-of-magnitude, per unit verified, in 2026 USD.

| Tier | Trigger | Mechanism | Cost / time per unit | What a pass proves |
| --- | --- | --- | --- | --- |
| V0 Locate | Any handling | QR / barcode scan → DID resolution | cents / seconds | Which record to check (nothing about the object) |
| V1 Consistency | Custody transfer, receiving | Visual + label vs. record fields; dual-signed custody event | <$1 / <1 min | Paperwork coherent; both parties committed to the claim |
| V2 Structural spot | Sampling at transitions; suspicion | Handheld EL against enrolled stable-structure template | $5–20 / minutes | This laminate matches enrollment at the checked cells |
| V3 Full structural | Adjudication, acquisition, catastrophe claims | Challenge-parameterized multi-modal verification (Ch. 6) | $50–300 / tens of minutes | Match across modalities and operating points; condition delta quantified |

Two features of the ladder do the economic work. First, the expensive tiers are sampled, and the sample is chosen by the verifier against a pre-committed design — so a lot of 1,000 modules gets V3 assurance at 30 × V3 cost, not 1,000 ×, with statistics carrying the rest. Second, each tier’s failure escalates to the next rather than to an argument: a V1 mismatch triggers V2 on the spot, a V2 miss triggers V3 and a ledger flag, and the escalation trail is itself evidence. Fraud, to succeed, must now survive every tier it might trigger, while honest assets almost never climb past V1 — an asymmetry of costs that lands, for once, on the right party.

A worked instance shows the ladder running. A broker receives a 22-pallet lot of used modules with ledger-documented histories. Receiving staff V0-scan every unit while unloading (forty minutes, resolving each QR to a DID and confirming the lot manifest) and V1-check the paperwork coherence — at which point two units resolve to DIDs whose records show state Decommissioned: an immediate, free catch of either paperwork error or laundering, impossible in a label-only world. The broker’s policy escalates the lot: a technician draws a V2 sample of 25 units by seeded random selection committed to the ledger before unpacking, and images them with a handheld EL rig against enrolled templates. Twenty-three match; two show template mismatch at the checked cells. Those two go to V3 — full multi-modal verification — which confirms one as a genuine match degraded by an undocumented repair (re-enrollment event filed, provenance discount applied) and one as a substituted unit (flagged, quarantined, dispute event opened against the seller). Total verification spend: a few hundred dollars against a six-figure lot, targeted almost entirely at the anomalies. The same lot under status-quo practice would have been priced blind, and both problem units would have shipped to a customer.

The composite pattern also answers a question that pure-mechanism thinking gets wrong: what happens when a mechanism fails honestly? Labels weather off; secure elements die; a template’s cells get shaded by new rooftop clutter. In a single-mechanism scheme each is an identity crisis. In the composite scheme each is a recoverable degradation: the surviving mechanisms re-anchor the identity, the failed one is re-provisioned or re-enrolled with a supersession event, and the incident leaves a record. Identity, once again, behaves less like a certificate and more like a thread of documented continuity — the theme Chapter 6 elevates to a design principle.

### 3.6.1 Identity for Aggregates: Strings, Plants, and Fleets

The chapter has treated the unit; deployments also need names for collections, and the design rule from Section 1.2.2 — full identity at the custody-and- liability level, aggregation upward as events — resolves most of the questions mechanically. A string is a wiring fact: it exists as installation-event payload (this module, this position, this string identifier) and changes by maintenance event; it needs no DID because nothing is ever warranted, sold, or adjudicated at string granularity that cannot be expressed over its members. A plant is different: it is a legal and commercial object — it holds interconnection agreements, is bought and sold as an entity, and is the subject of insurance policies — so it earns a DID of its own, whose relationship to its component assets is a time-varying membership recorded by installation and removal events. Plant-level verification then has a precise meaning: verify the plant DID’s record, then verify membership-consistency (every member’s record places it at this plant now), then sample members per the V-ladder. A fleet — an owner’s holdings across plants — is a portfolio view, not an identity: Chapter 10’s privacy analysis specifically wants fleet composition to be non-obvious from public data, so fleets exist as queries under the owner’s authorization, never as on-ledger objects. The general test for “does this collection get a DID?” is the same every time: does any external party ever need to verify a claim about the collection as such? Plants yes, strings no, fleets no — and battery packs versus cells, Chapter 12’s version of the same question, will use the same test.

## 3.7 Chapter Summary

Binding is the layer distributed ledgers cannot provide and the layer on which every guarantee in Part I ultimately rests, and this chapter has assembled its full apparatus. The verification function (V) is statistical (FAR/FRR under a threshold, published as a curve), adversarial (soundness priced in forger cost, never in collision chance), and instrumented (the verifier’s own attested instrument is part of the trusted base). Active binding — device keys in tamper-resistant hardware, verified by challenge–response — is mature, proven at national scale by the meter fleets, and fits every DER asset class that carries electronics, which is most classes by value and few by count; its lifecycle (provisioning ceremony, operational attestation, supersession at element replacement) mirrors the passive patterns closely enough that one verification logic serves both. The module, passive and numerous, requires the object itself to serve as credential: enrollment and re-verification of structural fingerprints through the acquisition–registration–extraction–comparison pipeline, of which electroluminescence structure is the practical present, structural defect mapping the high-assurance direction Chapter 6 develops, and depth-versus-accessibility the organizing trade. DIDs and verifiable credentials supply workable naming and claim formats once their person-centric assumptions — self-sovereign control, key rotation, indefinite resolution — are reworked for objects whose controllers change, whose “keys” are their physical structure, and whose records must verify without live resolution. Composition, not mechanism choice, is the deployment answer: locate cheaply, verify proportionally up the V0–V3 ladder, escalate failures rather than argue them, and grant collective identity only where external parties verify claims about the collective as such. The reference stack of Figure 3.2 — claims over names over records over bindings — is the architecture whose record layer Chapter 4 now designs in detail.

## References and Further Reading

- Grieves, M., and J. Vickers. “Digital Twin: Mitigating Unpredictable, Undesirable Emergent Behavior in Complex Systems.” In Transdisciplinary Perspectives on Complex Systems, 85–113. Springer, 2017. The origin of the twin concept in lifecycle management; read alongside the AAS specifications for the current industrial form.

- World Wide Web Consortium. Decentralized Identifiers (DIDs) v1.0. W3C Recommendation, 19 July 2022.

- World Wide Web Consortium. Verifiable Credentials Data Model v2.0. W3C Recommendation, 2025.

- Gassend, B., D. Clarke, M. van Dijk, and S. Devadas. “Silicon Physical Random Functions.” In Proceedings of the 9th ACM Conference on Computer and Communications Security (CCS ’02), 148–160. ACM, 2002.

- Buchanan, J. D. R., et al. “Fingerprinting Documents and Packaging.” Nature 436 (2005): 475.

- Trusted Computing Group. TPM 2.0 Library Specification. TCG, latest revision.

- Köntges, M., et al. Review of Failures of Photovoltaic Modules (EL imaging and crack morphology context). IEA-PVPS Task 13 Report T13-01:2014.

- Degen, C. L., F. Reinhard, and P. Cappellaro. “Quantum Sensing.” Reviews of Modern Physics 89, no. 3 (2017): 035002.

- Bundesamt für Sicherheit in der Informationstechnik. Protection Profile for the Gateway of a Smart Metering System (BSI-CC-PP-0073). Bonn: BSI. The metering-fleet precedent of Section 3.3.1.

- Plattform Industrie 4.0 / IDTA. Asset Administration Shell Specifications. The industrial digital-twin data models discussed in Section 3.2.