# Blockchain-Based Identity and Provenance for Distributed Energy Assets

Immutable Identity, Lifecycle Traceability, and Integrity Verification for Solar and Renewable Infrastructure

[AUTHOR NAME — TO BE SUPPLIED]

Table of Contents

# Extending the Model — Wind, Storage, and Broader IoT Renewable Assets

## What This Chapter Covers

The framework was developed on solar because solar is the hard case for volume and the module is the hard case for passivity. This chapter tests the framework’s claim to generality: it maps the identity, binding, and lifecycle machinery onto battery storage (where the EU passport regulation makes the exercise mandatory rather than speculative, and where a worked second-life transaction shows the whole apparatus running in its new dialect), wind-turbine components (where unit values are high, provenance disputes are mature, and a blade’s biography exercises the repair loop solar barely has), and the long tail of renewable-adjacent IoT hardware with its firmware problem and its aggregation-layer boundary. The organizing question for each asset class is the same triple the solar chapters answered: what is the unit of identity, what binds it, and which lifecycle events carry the value? — applied under an explicit test discipline that measures every class by the size of its diff against the base framework. The chapter then runs the framework’s edges (hydrogen, flow batteries, grid primary equipment, commodity BOS) to find where it stops paying, and closes with the interoperability problem — across asset classes, vendors, and jurisdictions — which deployment makes unavoidable and which the passport regulations are quietly deciding.

## 12.1 The Generalization Test

A framework generalizes if its abstractions carry over while only its parameters change — and the claim must be earned per class, because the graveyard of “general” architectures is filled with solar systems wearing battery costumes. The candidate abstractions from Parts I–III: the four identity properties (Section 1.2); active-versus-passive binding with the R1–R6 requirements (Sections 3.3–3.4, 6.5); the envelope-plus-state-machine event model (Chapter 5); tiered assurance (Section 6.7); and the three-zone privacy topology (Chapter 10). The claim to be tested class by class: only the binding modality and the event vocabulary’s payload schemas are asset-specific; everything else is configuration. Table 12.1 previews the result.

What the solar baseline actually consists of, recapped in one paragraph so this chapter’s diffs have their referent: unit identity at the module with composition downward and aggregation upward (Section 1.2.2, 3.6.1); EL-plus-defect-map passive binding under the challenge protocol for the passive population, secure elements for the active one (Chapters 3, 6); the eighteen-type event vocabulary on the nine-state machine with corroboration classes (Chapter 5); Merkle-batched writes, anchored consortium consensus, replicated custody (Chapters 4, 7); tiered assurance V0–V3 (Section 3.6); three-zone privacy with selective disclosure (Chapter 10); and the governance apparatus of accreditation, registers, and agreements (Chapters 8–10). Every clause of that sentence is about to be stress-tested by an asset that differs from a solar module in some load-bearing way.

The test’s method deserves a sentence, because generalization claims fail by hand-waving. For each asset class, the chapter answers the organizing triple concretely (unit of identity, binding mechanism against R1–R6, value-bearing events), checks each answer against the class’s economics per Section 1.3’s discipline, and reports the deltas — every place the class forced an addition or an exception — because a framework’s honest measure is the size of its diff, not the enthusiasm of its abstract. The solar chapters’ machinery is taken as the base; a delta that touches only payload schemas and added event types passes; a delta that would touch the envelope, the state machine’s existing transitions, or the verification workflow’s structure fails, and the chapter says so where it happens (cell-level binding being the standing failure). The chapter’s route: batteries first and longest (Section 12.2), because regulation has made them the forcing case; wind (12.3) for the high-value, repair-loop inversion; the IoT tail (12.4) for the firmware dimension; the interoperability problem and its venues (12.5); and the framework’s edges and outward export (12.6), where the honest boundary-drawing lives.

Table 12.1 The framework mapped across asset classes. Bold entries mark the chapter’s two structural findings: the open cell-identity problem and the one asset class whose adoption clock is set by statute.

|  | PV module | Battery pack/cell | Wind blade/bearing | Inverter-class IoT |
| --- | --- | --- | --- | --- |
| Unit of identity | Module (cell optional) | Pack; module; cell is the open question | Component (blade, bearing, gearbox stage) | Device |
| Binding | Passive: defect map / EL (Ch. 6) | Active at pack (BMS); passive electrochemical at cell | Passive: material/structural signatures + embedded tags | Active: secure element |
| Hardest lifecycle stage | Secondary resale | Second life & chemistry drift | Repair provenance | Firmware churn |
| Value-bearing events | Commission, condition, transfer | SoH records, repurposing, recycle (mandated) | Repair/inspection, load-history milestones | Attestation, config change |
| Regulatory driver | ESPR candidate | Battery Reg. 2023/1542 — live | Type-cert regimes | Grid codes (Ch. 10) |
| Framework deltas | — (the baseline) | SoH payload schemas; repurposing event pair | Load-milestone events; repair sub-DAG | Config-change events; attestation cadence |

Reading the table’s rows against each other yields the chapter’s comparative anatomy. The unit of identity row shows the custody-and- liability rule of Section 1.2.2 producing different answers from the same principle — module, pack, component, device — with the open question (cells) flagged rather than fudged. The binding row is the R1–R6 requirements meeting four different physics, and its pattern is instructive: binding difficulty tracks passivity and unit economics, not asset sophistication — the humble module remains the hardest case in the book. The hardest-stage row locates each class’s fraud economics where its information asymmetry peaks. And the regulatory driver row explains adoption sequencing better than any technology argument: batteries move first because law moved first.

## 12.2 Battery Storage: The Regulated Case

The regulatory context first, since it reorders every priority. The EU Battery Regulation entered into force in 2023 with obligations phasing through the decade: carbon-footprint declarations, due-diligence duties, and — from February 2027 — the digital passport for EV, light-transport, and industrial batteries above 2 kWh, hosted by the economic operator, carried via a unique identifier on the battery, with data obligations running through repurposing and recycling actors to end of life. Analogous frameworks are moving in other jurisdictions at varying speeds, and the global battery industry, being global, will in practice build to the strictest common denominator. For this book the consequence is blunt: the record-keeping argument of Parts I–II is settled by statute for this asset class, and the open question is only whether the mandated records will be worth anything — which is the verification question the architecture answers.

Batteries invert two solar assumptions, and the inversions are instructive — the first about what the binding can anchor to, the second about which lifecycle paths carry the money.

The asset is chemically alive. A module’s identity anchor is structure that manufacturing froze; a cell’s electrochemistry evolves by design with every cycle. The stable/condition decomposition of Section 6.3 still applies but the balance shifts: less of the measurable signature is stable, and the condition layer becomes the commercially dominant record — state of health is the price of a used battery. Binding follows Section 3.3 at pack level (the BMS is a computer; secure-element identity is straightforward and increasingly mandated in effect by passport data requirements) — while cell-level identity remains genuinely open: cells are numerous, passive to the outside, and repackaged during refurbishment precisely when identity matters most. Candidate passive anchors (impedance- spectroscopy signatures, formation-process fingerprints) currently fail R2 or R6 of Table 6.2, and this book flags rather than solves it (→ Chapter 14). The failure modes are worth one diagnostic sentence each, since they define the research target: impedance signatures carry genuine per-cell individuality (electrode microstructure, electrolyte wetting) but the features drift with exactly the cycling the identity must survive — an R2 failure unless a cycling-invariant subspace can be isolated, which is open problem M4’s precise statement; formation-process fingerprints (the first-cycle voltage traces manufacturers already record) are stable records but not re-measurable properties — verification would compare a stored curve against a stored curve, which authenticates the paperwork, not the cell (the Section 1.2.1 distinction, biting again); and optical or structural approaches face the R6 wall that cells live inside welded packs, measurable only at manufacture and teardown. A modality that survives cycling, re-measures through the pack, and prices at cell economics would close the gap; nothing published does all three. The honest interim: pack-level binding, cell-level composition manifests committed at manufacture and re-attested at every repackaging, so that cell substitution is at least a Class-C attributable event rather than an invisible one.

The SoH record deserves the same oracle discipline the solar chapters gave condition imaging, because it is where the battery money argues — a used pack’s price is its SoH claim multiplied by trust in the claimant, and the second factor is what this architecture manufactures. State of health is not a measurement but an estimate — a model over capacity tests, impedance, and cycle history, sensitive to temperature, rest state, and the estimator’s own algorithm — so the schema treats an SoH attestation the way Chapter 6 treats a template: the claim carries its method (protocol version, pre-conditioning state per Section 8.4’s battery-spoofing note, the raw measurement digests behind it), and verifiers weight SoH numbers by the method’s grade rather than treating all percentages as equal. The BMS’s own lifetime telemetry — cycle counts, temperature histograms, abuse-event flags, signed under the pack’s element identity — enters as the aggregate-then-commit pattern borrowed from wind (Section 12.3): periodic usage-milestone events whose payloads summarize the operational history that SoH models consume. The composition works: an attested milestone trail plus a method-graded SoH measurement at transition is a second-life valuation input that no party can quietly improve, which is precisely the missing instrument of Section 1.4.4’s battery lemons problem.

Safety events add a battery-specific record class with regulatory teeth — and with the strongest incentive asymmetry in the whole chapter: thermal incidents, abuse events flagged by the BMS, and transport- classification changes (a damaged pack becomes dangerous goods with different carriage rules) all demand the same contemporaneous, attributed, non-repudiable treatment the schema gives everything else — and the incident record travels with the pack’s identity into every downstream decision, because the party least likely to disclose a thermal event voluntarily is exactly the party selling the pack. Insurers of second-life installations have begun asking for precisely this record; the schema’s EVT_FAULT class with BMS attestation carries it without amendment.

The lifecycle flows themselves are more branched than solar’s, and the event vocabulary must follow the branches. An EV pack retiring from vehicle service forks four ways: direct second-life deployment (the repurposing pair below), remanufacture (disassembly to modules or cells, reassembly into new packs — which is where cell-level identity’s absence bites hardest, and where the composition-manifest discipline earns its keep: the new pack’s manifest references the donor packs’ DIDs and their milestone trails, preserving lineage even where per-cell binding fails), materials recycling (the mandated mass-balance terminus), and warranty return to the OEM (a custody loop, not a terminus). Each fork is an event pattern already in the schema’s grammar; what batteries add is the frequency with which assets take them, and therefore the commercial weight on the paths solar treats as edge cases — a pack is likelier to be remanufactured than a module is to be resold, and the schema’s traffic statistics will show it.

The actor map differs from solar’s in ways that reshape consortium design, and getting it on paper early is the battery deployment’s Phase-0 homework. Battery custody chains run through automotive OEMs (whose warranty and recall machinery dwarfs solar’s), fleet operators, dismantlers and authorized treatment facilities, repurposers (a new legally-defined actor class the regulation created), integrators, and recyclers under mandatory take-back — with the automotive OEM holding both the manufacturing records and, through the vehicle relationship, the usage telemetry that solar’s fragmented chain never concentrated in one place. The consortium-composition consequence: battery federations will orbit the automotive OEMs’ gravitational mass, and the adverse-interest discipline of Section 7.5 has to be constructed deliberately (independent repurposers, insurers, and recyclers seated with real weight) where solar’s fragmentation provided it for free. The concentration also simplifies one hard solar problem: retroactive enrollment of the existing pack fleet is largely an OEM database export plus re-attestation, not a field campaign — the records exist; they need anchoring and portability, which is the cheaper half of this book.

The battery section’s forecast, stated for Chapter 14 to grade: the first production-grade deployment of this book’s full architecture — binding, events, anchoring, verification services — will be a battery deployment, driven by the passport deadline, organized around an automotive OEM’s compliance program, with its verification layer retrofitted onto a records system built first for the regulation’s minimum. The architecture was designed so that this retrofit is a projection-layer inversion rather than a rebuild, and the design’s market test is whether that proves true.

The lithium supply chain’s upstream — mining, refining, precursor and cathode production — extends the composition-reference machinery (Section 4.3) into the territory the due-diligence articles regulate: each stage’s attestations chain into the cell batch’s manifest, so the pack’s passport can substantiate origin claims at the depth the regulation demands without any party holding the whole chain’s data. The caveat from Section 10.4 travels unchanged: the chain authenticates declarations, and mineral-origin fraud upstream of the first attested stage remains a procedural problem for audit regimes — the ledger sharpens the paper trail’s teeth; it does not visit the mine.

Second life is a first-class path, not an edge case. The solar state machine needed Removed → InTransit for resale; batteries need a richer structure: a repurposing event pair (EVT_REPURPOSE_OUT from vehicle service, EVT_REPURPOSE_IN to stationary service) carrying mandatory SoH payloads, because the EU regulation makes repurposers legally responsible parties and the passport must document the transition. Figure 12.1 extends Figure 5.1 accordingly — the extension is additive: no solar state or transition changed, which is the generalization claim passing its first test. Reading the figure: the second-life states are the base machine’s states with an application-context annotation, not new lifecycle physics — an asset in “Commissioned (2nd life)” accepts exactly the base state’s events, and a verifier’s traversal crosses the repurposing boundary as one more edge in the DAG, inheriting the first life’s history rather than starting a second file. The one deliberate asymmetry: there is no arrow back from second-life states to first-life application classes, because a pack does not return to vehicle service under any regulation now drafted, and the state machine encodes the prohibition rather than trusting policy to remember it.

Figure 12.1 Battery lifecycle extension to the Chapter 5 state machine. Unshaded states are inherited unchanged from Figure 5.1; the figure shows only the extension’s neighborhood, since everything upstream of the first Removed is the base machine verbatim.

stateDiagram-v2
    state "Installed (2nd application)" as Installed2
    state "Commissioned (2nd life)" as Commissioned2
    state "Removed (2nd)" as Removed2
    Commissioned --> Removed: EVT_REMOVE
    Removed --> Repurposing: EVT_REPURPOSE_OUT (SoH payload, Class C)
    Repurposing --> Installed2: EVT_REPURPOSE_IN (new application class)
    Installed2 --> Commissioned2: EVT_COMMISSION
    Commissioned2 --> Removed2: EVT_REMOVE
    Removed2 --> Decommissioned: EVT_DECOMMISSION
    Decommissioned --> Recycled: EVT_RECYCLE (mass balance mandated)

The passport is a projection, and the projection was already designed. Section 10.4 argued the event schema must project onto passport data models; for batteries this is now a compliance exercise with a deadline. The mapping is mechanical — passport identity attributes from EVT_REGISTER, performance and durability fields from condition records, supply-chain due-diligence attestations as VCs, access tiers onto the three-zone topology — and the read-side mapping layer of Section 10.4 becomes the passport interface. Table 12.3 exhibits the mapping’s shape for the regulation’s principal field groups.

Table 12.3 Battery-passport field groups (Regulation 2023/1542, Annex XIII) and their sources in the event schema.

| Passport field group | Schema source | Verifiability upgrade over self-declaration |
| --- | --- | --- |
| Identity, model, manufacturer | EVT_REGISTER + manufacturer VCs | Anchored issuance; registrar accreditation |
| Composition, materials, hazardous substances | Composition manifests + upstream VCs (§4.3) | Attributable supply-chain chain-of-claims |
| Carbon footprint declaration | Manufacturer VC with method reference | Method-graded, contestable, dated |
| Performance & durability (capacity, SoH, expected life) | Method-graded SoH attestations + usage milestones | Instrument attestation; pre-conditioning recorded |
| Circularity: repair, repurposing, dismantling info | Repair/repurposing events + documentation payloads | Events, not brochures — history, not intentions |
| Supply-chain due diligence | Due-diligence VCs per reporting period | Anchored, versioned, auditable |
| Labels & certifications | Certification VCs | Revocation-visible (§3.5.1) |

What the regulation does not supply is exactly what this book’s architecture adds: verification. A passport whose SoH field is a self-declaration reproduces Chapter 1’s problem in a new format; a passport whose fields resolve to anchored, instrument-attested, corroborated events is the difference between paperwork and evidence. This is the book’s thesis restated in regulatory clothing, and it is why battery deployments are likely to adopt the architecture before solar ones do: the record-keeping is already mandatory, so the marginal cost of making it verifiable is small.

A second-life transaction, walked end to end, assembles the battery machinery the way Section 11.3’s scenarios did solar’s — and it is the transaction the passport regulation’s drafters had in mind, so consider it the compliance regime’s own use case, run properly. A fleet operator retires 400 EV packs; a storage integrator bids for them. Under the passport-minimum world, the integrator receives 400 self-declared SoH percentages and prices for the worst case (the Section 1.4.4 lemons discount, unchanged by the passport’s existence). Under the full architecture: each pack’s DID resolves to its milestone trail (BMS-signed usage history), its retirement EVT_REPURPOSE_OUT carries a method-graded SoH measurement performed under recorded pre-conditioning by an accredited facility, and the integrator’s client runs Steps 1–2 across the lot in an afternoon — then draws a pre-committed sample of twenty packs for independent SoH verification at its own bench (Step 3, battery dialect). The measured spread against the attested values becomes the lot’s price adjustment, mechanically; the packs re-enter service with EVT_REPURPOSE_IN events binding their first and second lives into one history; and the integrator’s own financing — this is the step that closes the loop — cites the verified trail, because its lender’s technical advisor consumes the same records. Every mechanism is a solar mechanism with battery nouns; the delta remains payloads and two event types. The transaction also names the service industry batteries will need before solar does: accredited SoH verification facilities — the battery counterpart of Section 6.7’s verification-services tier — whose bench capacity, accreditation regime, and method proficiency testing are currently improvised per deal. The passport deadlines will industrialize this function within the decade; the open question is whether under the metrological discipline this book keeps advocating or as another archipelago of proprietary methods.

## 12.3 Wind: High Value, Low Volume, Mature Disputes

Wind inverts solar’s economics: a blade set or main bearing carries the value of thousands of modules, unit counts are in the tens of thousands rather than hundreds of millions, and the provenance disputes are already institutionalized (serial-number confusion across repair shops, undocumented blade repairs surfacing in resale and insurance, gearbox refurbishment chains). The sector’s documentary culture is correspondingly older and thicker — type-certification regimes, per-component quality dossiers, inspection-report chains — which changes the pitch: wind does not need persuading that component history matters (its insurers settled that argument decades ago); it needs the Chapter 1 diagnosis applied to records it already keeps expensively — scattered across OEM, operator, and repair-shop systems, keyed by serials of varying reliability, and reconstructed manually at every transaction. The framework arrives in wind as consolidation and verification of an existing practice rather than the creation of one, which lowers adoption friction and raises the integration bill in equal measure. The framework maps with four adaptations:

- Component-level DIDs with an assembly DAG. The turbine is a long-lived assembly whose components individually detach for repair and return — or don’t return. The prior_refs DAG (Section 5.4) already expresses composition; wind makes it the primary structure: a nacelle’s identity is a slowly-changing graph of component identities, and the valuable query is “show me this bearing’s custody and repair history across its three host turbines.” The aggregate-identity test of Section 3.6.1 gives the turbine itself a DID (external parties verify claims about the turbine as such — certification, insurance, repowering sales), while the tower sections and fasteners stay compositional; the interesting boundary case is the gearbox, which trades as a unit through refurbishment pools and therefore earns full identity despite living its whole life inside assemblies. The refurbishment pool is worth dwelling on because it is the sector’s standing identity crisis: a failed gearbox ships to a rebuild shop and an equivalent — rebuilt from the pool’s parts stock, possibly containing components from a dozen donors — ships back, with the operator’s records typically tracking the exchange as a repair rather than the component swap it is. The framework’s answer is the remanufacture pattern from batteries (Section 12.2): the returned unit is a new identity whose composition manifest references its donor components’ DIDs and their load histories, so the operator’s fatigue accounting runs on the actual internals rather than the nameplate — a change the insurers of gearbox failures, the sector’s costliest recurring claim class, have every incentive to demand.

- Passive binding is easier here, and cheaper options suffice. Blades are large composite structures with rich manufacturing individuality (ply layup texture, cured-resin signatures readable by ultrasonic or thermographic scan) and — decisive difference — per-unit values that justify embedded secure tags at manufacture. R3’s forgery asymmetry is comfortable; the binding research frontier that Chapter 6 needed for modules is optional insurance for blades. The R-dossier, run quickly for the record: entropy (R1) is abundant in layup and cure stochastics; the stable subset (R2) is friendlier than solar’s because composite internal structure, barring damage, does not evolve in service the way crack networks do; challenge parameterization (R4) is available through multi-angle ultrasonic response; evolution plausibility (R5) maps onto the well-developed composite damage- progression literature; and field feasibility (R6) rides the ultrasonic inspection regime blades already undergo. Only enrollment logistics differ in kind: a 70-meter blade is fingerprinted in sections, with the section map part of the template.

- Turbine controllers are the IoT tail at nacelle scale. The config-change class of Section 12.4 applies verbatim — a turbine’s control firmware determines its loads, so the firmware history is fatigue-relevant, coupling the software record to the structural one in a way no other class exhibits — and F12’s dual-authorization discipline matters at proportionally higher stakes.

- Load history milestones as events. A blade’s remaining life is a function of its load history, which lives in high-volume SCADA streams (off-chain, Table 4.1’s logic unchanged). The adaptation: periodic load-milestone events — attested aggregates (equivalent-fatigue-cycle counters) committed at inspection boundaries — so that resale and insurance decisions have anchored fatigue evidence without the ledger touching telemetry. This pattern (aggregate-then-commit at decision boundaries) is wind’s contribution back to the general framework; it retrofits directly onto battery cycle-count and solar soiling-loss records.

A blade biography, compressed, to show the machinery running on wind’s characteristic path — the repair loop that solar barely has, and the path on which the sector’s undocumented-repair disputes have traditionally been fought. Blade B-7741 enrolls at manufacture (sectioned ultrasonic template, embedded tag, materials manifest), installs on turbine T-112, and accumulates load-milestone events at each annual inspection. Year nine: lightning damage; EVT_FAULT with drone imagery; the blade ships to a repair facility — custody transfers, a repair event class (wind’s main vocabulary addition) carrying the repair scope, materials, and post-repair ultrasonic re-verification whose comparison against the enrollment template both confirms identity and documents the repair’s structural footprint; the blade returns to a different turbine, T-089, whose assembly DAG updates. Year seventeen, repowering: the blade’s documented fatigue budget — milestone trail plus two repairs — prices it for a secondary deployment in a lower-wind market rather than the scrapyard, the same deadweight-rescue economics as solar’s Section 1.4.4, at three orders of magnitude higher unit value. Every event in the biography used base-schema machinery except the repair class and the milestone aggregates: a two-item delta for a sector whose provenance disputes predate solar’s entire existence.

Offshore wind sharpens every number in the onshore story — and supplies the framework’s best cost-benefit ratio anywhere in this chapter — while adding one structural wrinkle. Component values multiply (installation vessels cost more per day than most repairs cost outright, so avoiding one unnecessary jack-up campaign can justify a plant’s entire identity infrastructure); inspection access is scarce and weather-gated, which raises the value of remote Step-1/2 verification precisely because Step-3 visits are gold; and the marine environment accelerates the degradation phenomena the condition records track. The wrinkle is jurisdictional: offshore assets sit in exclusive economic zones under layered regulatory regimes, their components rotate through onshore repair pools serving farms in multiple countries, and the federation topology (Section 7.4) meets its most demanding test — a bearing’s custody chain can cross three ledgers in one repair cycle, exercising the cross-federation traversal at a frequency onshore fleets never will.

Institutionally, wind’s adoption path runs through different doors than solar’s: the OEMs are few, vertically powerful, and already operate proprietary component-tracking systems — so the realistic first deployment is an OEM opening its existing records to the verification layer (anchoring, third-party proofs) rather than a consortium building from zero, with the independence problem (Chapter 1’s vendor-cloud critique) addressed by anchoring and portability obligations negotiated into service contracts by the operators and insurers who hold the commercial leverage. It is a different route up the same mountain, and the framework’s platform-neutrality (Section 2.6) exists precisely so that both routes arrive at interoperable records. The sector’s data politics are the route’s real gradient: turbine operational data is already a contested asset between OEMs (who want service-contract exclusivity) and operators (who want maintenance-market competition), with regulators increasingly interested in the competition question — and the identity layer, which needs only sparse evidential extracts, offers both sides a settlement surface: the operator gets portable, verifiable component histories that make third-party maintenance biddable; the OEM keeps its high-frequency telemetry moat. Whether the parties take the settlement is commerce, not architecture; the architecture’s job was to make it available.

## 12.4 The Long Tail: Inverter-Class IoT and the Firmware Problem

The long tail’s population deserves a moment’s enumeration before its common problem, because its diversity is the point. EV chargers (tens of millions of units, commerce-critical, publicly sited); smart meters (the solved precedent of Section 3.3.1, now a component population inside this framework); residential and C&I gateways and energy managers (the orchestration layer of the behind-the-meter world); tracker controllers and the emerging robotic-cleaner fleets (mobile assets whose own custody histories matter); heat-pump and EV-charging controllers in the electrified-heat segment. All share the profile: active binding per Section 3.3, lifecycle per Chapter 5, nothing new — except the one dimension solar underweights: mutable software on a fixed device. For these assets the config/firmware state is safety- and grid-relevant (IEEE 1547 settings, inverter grid-support curves), changes hundreds of times over a service life, and is invisible to physical binding. The adaptation: a EVT_CONFIG_CHANGE event class carrying firmware/config digests signed by the device’s secure element and the party commanding the change, giving the DSO of Section 10.4 a verifiable answer to “what code, what settings, since when” — the device-identity equivalent of the calibration chain instruments already have (Section 4.5, Rule 3). The threat model gains F12, malicious-update-with-valid- keys, which is bounded the same way F10 was: dual authorization and attributability, not cryptographic prevention.

The config-change class needs volume discipline or it swallows the evidential tier: a national charger fleet’s routine update campaigns would generate millions of identical events. The answer is the batching grammar already built — campaign-level events committing a Merkle root over per-device confirmations, each device’s inclusion individually provable (Section 7.2’s pattern, third reuse) — plus a severity taxonomy: grid-behavioral changes (protection settings, support curves) are evidential events per device; cosmetic firmware revisions travel as campaign aggregates. The DSO subscription then filters at exactly the severity boundary its codes care about.

The aggregation layer above the devices — virtual power plants, demand- response fleets, flexibility markets — deserves its boundary statement, because it is where this framework most often gets over-sold. An aggregator’s settlement records, dispatch histories, and baseline calculations are market data with their own integrity regimes; the asset-identity layer’s contribution is the substrate those regimes currently lack — verifiable device identity, certified capability (config-change history), and location class for the fleet an aggregator claims to control. “Which devices, with what certified behavior, does this VPP actually command?” is an asset-identity question this architecture answers — and one that flexibility-market rules increasingly require aggregators to answer credibly at prequalification; “did the VPP deliver its flexibility bid?” is a market-settlement question it deliberately does not. Deployments that keep the boundary keep both systems honest; those that blur it inherit the energy-trading scope creep Section 1.6 excluded on page one.

Two class-specific notes complete the long tail. EV charging infrastructure adds a commercial dimension the other classes lack: chargers are metrologically certified billing instruments, so their identity records intersect legal metrology (the meter-fleet precedent of Section 3.3.1 applies nearly verbatim — Germany’s charging-calibration regime already mandates signed meter records per session) and their config-change history is also a tariff-integrity record — the regulator’s question is not only “is the grid behavior certified?” but “was the customer billed by the firmware the certificate covers?” Heat pumps and behind-the-meter controllers stress the privacy design hardest: their operational patterns are household telemetry of the most intimate kind, and the architecture’s discipline — evidential events sparse and appliance-focused, operational streams never leaving the home platform, Chapter 10’s zones enforced at the schema — is the difference between an asset registry and a surveillance system wearing one’s clothes. The residential interface-austerity note of Section 10.2 was written for solar and applies here with the volume turned up.

The long tail’s conformance economics close its treatment: these device classes ship in millions from dozens of vendors into certification regimes (metering directives, charging-infrastructure regulations, appliance standards) that already operate type-approval machinery. The framework’s asks — secure elements, attested firmware, config-change events — are cheapest to impose inside those existing regimes as certification criteria rather than beside them as consortium requirements, which is the meter fleets’ lesson (Section 3.3.1) generalized: where a certification chokepoint exists, use it; the open market’s long tail is exactly the population for which voluntary consortium adoption moves slowest and regulatory reference moves fastest.

## 12.5 Interoperability: The Problem Deployment Forces

A fund owns solar, storage, and wind; a recycler receives all three; an insurer underwrites the portfolio. If each asset class — worse, each vendor — carries its own identity scheme, the verification cost the architecture eliminated returns as integration cost — and the cross-class actors are not edge cases but the value-bearing cases: Chapter 13 will show that insurers, lenders, and recyclers, the parties whose verification demand drives adoption, are all portfolio parties by construction. Interoperability is therefore not a phase-two nicety; it is the adoption engine’s fuel line. Three interoperability layers, in order of difficulty:

Table 12.2 Interoperability layers and their current state. The rows order by difficulty is also their order of consequence: an envelope mismatch costs an adapter; a vocabulary mismatch costs analytics; a binding mismatch costs the ability to verify at all.

| Layer | Requirement | Instrument | State of play |
| --- | --- | --- | --- |
| Envelope | One event envelope, one DID resolution path across classes | This book’s schema; W3C DID/VC | Achievable now by design discipline |
| Payload semantics | Shared vocabularies per domain (SoH, fatigue, degradation) | Passport delegated acts; IEC TC82/TC88/TC21 work | Fragmentary; passports forcing convergence |
| Binding verification | Cross-vendor template & threshold standards (§11.7 problem 2) | Does not exist | The genuine gap (→ Ch. 14) |

The envelope layer is the argument for the federated topology one more time: class- or jurisdiction-specific consortium ledgers, mutually anchored, sharing the envelope and DID method — so a portfolio verifier runs one Step-1/Step-2 client against all of them (the third independent requirement to land on federation, after scalability in Section 7.4 and data residency in Section 10.4; by now the pattern has earned its place as the default deployment shape). The payload layer is being settled, de facto, by the EU passport delegated acts — whoever writes those data models is writing the sector’s shared vocabulary, a point Chapter 14 makes with some urgency for the PV act still in motion. The binding layer has no standards body engaged at all, and it is where this book’s research community has the most to contribute and the least time to do it: once large fleets enroll under proprietary template formats, the switching costs will entrench whatever shipped first.

The portfolio verifier’s day, walked once, makes the interoperability stakes concrete — and every reader who manages mixed-asset infrastructure will recognize the translation layer in it, because they are paying for one now under a different name. An infrastructure fund’s monitoring client wakes to verify overnight events across its holdings: solar plants on two national federations, a battery portfolio on a storage-sector consortium, wind assets on the manufacturer-led ledger their turbines shipped with. At the envelope layer, one client verifies all four — same DID resolution, same proof machinery, same anchor checks — if the S1/S2 layers hold; today, with the wind vendor’s ledger using its own event grammar, the fund runs two clients and a translation layer its auditors accept grudgingly. At the payload layer, the fund’s degradation dashboard wants solar condition records, battery SoH attestations, and blade fatigue milestones in comparable form; the passport data models will force the convergence the analytics vendors have not. At the binding layer, the fund’s diligence playbook for acquisitions currently specifies verification procedures per vendor ecosystem — the exact per-relationship cost Chapter 1 diagnosed, reborn one level up. Every layer’s gap is somebody’s integration invoice, and the invoices, not the standards essays, are what will drive the convergence.

### 12.5.1 Who Convenes the Convergence

Naming the venues, because “standards should exist” is a sentiment and committee calendars are facts. The envelope and DID method work (S1/S2) has natural homes in ISO/TC 307’s DLT program crossed with the sectoral committees — IEC TC 82 (solar), TC 21 (batteries), TC 88 (wind) — with the practical path running through a joint working group anchored by whichever sector’s consortium ships a working profile first; running code has always outvoted position papers in that room. The payload vocabulary is being convened whether the sector likes it or not by the European Commission’s delegated-act machinery with CEN/CENELEC JTC 24 drafting the passport framework standards — which is why this book keeps urging the measurement community to show up there: the acts will cite whatever data models exist when their deadlines arrive. The binding layer has no venue, and Section 14.3’s proposal (metrology institutes hosting reference template formats and proficiency testing) needs a first mover; the concrete suggestion this book can make is that the IEA-PVPS task structure — which already coordinates the international measurement community that would do the work — commission the gap analysis, because its reports have a track record of becoming IEC work items. Industry alliances (the battery-passport consortia, the DER interoperability alliances) matter as profile builders and pilot conveners rather than standards authors — their histories suggest they move fastest and bind least, which is the correct division of labor with the treaty-grade bodies. The convergence’s clock, one last time: fleets are enrolling now, delegated acts are drafting now, and every year of venue-shopping is a year of proprietary formats compounding their switching costs — the S3 window (Section 14.3) does not wait for the committee that would prefer to be thorough.

Cross-sector event semantics deserve their own caution flag before the section closes: the same word means differently across classes, and naive vocabulary sharing manufactures confusion. “Commissioning” starts a solar warranty, but a battery’s warranty typically keys on first charge at the integrator; “capacity” is nameplate DC for a module, usable energy under a test protocol for a pack, and a wind-class rating at a reference speed for a turbine. The schema’s answer is namespacing rather than false unification — event types carry their class vocabulary, projections onto shared reporting models (passports, portfolio dashboards) do the translation explicitly and versionably — and the S1 standardization work should resist the committee temptation to define one “commissioning” for all assets, which would be wrong for most of them.

## 12.6 Beyond the Big Three: The Framework’s Edges

Generalization claims are tested at their edges, so the chapter closes by running the triple over the asset classes the earlier sections did not reach — and by stating plainly where the framework stops paying, because an architecture that claims to fit everything fits nothing in particular.

Electrolyzers and fuel-cell systems map almost boringly well: high unit values, active binding trivially available (they are instrumented process plants), stack degradation as the condition record, and an emerging certification demand — renewable-hydrogen guarantee-of-origin schemes, whose additionality and temporal-correlation rules need verifiable equipment identity and commissioning dates as inputs — that needs exactly the equipment truth layer of Section 10.4. The delta is payload vocabulary (stack hours, degradation curves, electrolyte chemistry) and nothing structural; the stack-replacement cycle even reuses the battery remanufacture pattern without alteration. Flow batteries and long-duration storage split cleanly along the framework’s existing seam: the power-conversion and stack hardware follow the electrolyzer pattern, while the electrolyte inventory — a bulk fungible asset, tanker-delivered and blended — does not want unit identity at all but lot provenance, the composition-manifest pattern at tank scale. The electrolyte is the framework’s fungibility lesson in one object: vanadium electrolyte retains value across plant generations and is already leased as a commodity, and a leased, blended, value-bearing fluid is tracked the way custody systems have always tracked fungibles — by lots, transfers, and mass balance — with the ledger adding attributability to a practice, not identity to molecules. Transformers and grid primary equipment are the confounding easy case: high value, low count, stationary, already serialized and registry-tracked by utilities — the framework fits but adds least, because Section 1.3’s distinctive stressors (volume, churn, custody fragmentation) are absent; the honest pitch there is narrow (supply-chain integrity for new procurement, given documented counterfeit-component concerns in grid equipment, plus the composition-manifest treatment of bushings and tap changers, whose counterfeits are the documented entry point) rather than lifecycle-total. Where the calculus shifts is the mobile substation and spare-transformer pools that grid-resilience programs increasingly maintain — shared, custody-churning assets that inherit the sector’s stressors and, with them, the framework’s full case. Mounting structure, cabling, and commodity BOS sit below the identity floor: unit values in single dollars, failure modes collective rather than individual, and no transaction that ever asks about one bracket. They get batch identity through composition references, and the framework’s discipline is to resist enthusiasts who would enroll every bolt — an identity system’s credibility includes knowing what not to identify.

Emerging PV chemistries belong in the edges section rather than the solar chapters, because their identity story is genuinely unfinished. Perovskite and tandem modules will arrive with the same passive-binding needs as silicon and none of its accumulated defect phenomenology: the stochastic individuality is certainly present (solution-processed layers are nothing but process stochastics), but which features are stable (R2), how they respond to the challenge protocol (R4), and what legal evolution looks like (R5) are open dossiers — Section 6.8’s problem 5, restated as a deployment gate. The practical sequencing writes itself: these products will launch under accelerated-degradation scrutiny anyway, and the instrumented-cohort programs their qualification already requires are the R-dossier studies wearing different funding labels; the marginal ask is that they enroll their cohorts under the framework’s template discipline from the first wafer, so the binding science and the product mature together rather than a decade apart, as silicon’s did.

Microgrids and energy communities, finally, test the aggregate machinery rather than any binding: a microgrid is a plant-like legal object (it holds interconnection and islanding agreements — it passes the Section 3.6.1 test and earns a DID) whose membership is unusually fluid, composed of assets under many small owners, and whose regulatory forms (the EU’s energy-community directives, community-solar programs in US states) are multiplying faster than their record-keeping practices. The framework handles it as a membership register with consent-aware events — an asset’s enrollment into a community is a dual-signed event like any custody act — and the privacy design of Chapter 10 does the heavy lifting, since community composition is precisely the kind of household-adjacent data the three-zone model exists to protect.

The general boundary rule, extracted from the edges: the framework pays where unit value × information asymmetry × custody churn clears the enrollment-plus-governance cost, and the product is a better screen than any single factor. Batteries clear it spectacularly (all three factors high); transformers clear on value alone but with thin margins of benefit; brackets never clear it. Classes that fail the screen still inherit protection indirectly — as composition references inside assets that pass — which is the framework’s graceful floor, and the reason its edges are boundaries rather than cliffs.

One final generalization runs outward rather than across — a paragraph this book permits itself after a volume’s worth of restraint: nothing in the framework’s core is energy-specific. Manufactured assets with stochastic structure, multi-decade lives, fragmented custody, and value-bearing histories exist in construction equipment, rail rolling stock, medical devices, and aerospace’s cost-pressured lower tiers — and each has its own version of Chapter 1’s memory problem. The energy sector’s claim to go first rests on the conjunction this book has traced (regulatory mandates arriving, unit economics forcing the hard passive-binding research, decarbonization stakes funding the institutions); the export of the resulting patterns to adjacent industries is left as the literature’s exercise, with the R1–R6 requirements and the value-screen rule as the transferable core.

## 12.7 Chapter Summary

The generalization test passes with informative strain, measured in diffs against a solar baseline recapped at the chapter’s head so every delta had its referent. Batteries keep the whole apparatus but shift its center of gravity from identity toward condition — pack-level active binding is easy; cell-level binding fails R2 or R6 in every published modality, with the failure modes now stated precisely enough to aim research at; SoH enters as a method-graded estimate under the full oracle discipline; the lifecycle forks (second life, remanufacture, recycling, warranty return) all parse in the base grammar; and the EU passport regime converts the architecture from proposal into compliance infrastructure with a verification upgrade whose field-by-field mapping Table 12.3 exhibits and whose commercial meaning the worked second-life transaction demonstrates. Wind keeps the apparatus and relaxes its economics — component DAGs with the gearbox as the instructive boundary case, blade fingerprints whose R-dossier reads easier than solar’s, load-milestone aggregates, and a repair loop whose two-item delta serves a sector whose provenance disputes predate solar’s existence. The IoT tail contributes the firmware problem, its config-change event class with campaign-scale batching, the legal-metrology intersection at chargers, and the aggregation-layer boundary that keeps this framework out of the market-settlement business. The edges — hydrogen, flow batteries, grid primary equipment, commodity BOS — locate the value screen (unit value × information asymmetry × custody churn) below which identity is composition reference, not enrollment. Across all classes the deltas were payload schemas, a few event types, and binding modalities — the envelope, state machine, tiering, zones, and governance carried unchanged, which is what “framework” is supposed to mean. The unresolved frontier is interoperability’s third layer, cross-vendor binding verification, where standardization has not begun and where Section 12.5.1 names the venues that could begin it. And the framework’s core, tested across four asset classes and its own edges, turns out to owe nothing to energy at all — the export to other manufactured-asset industries waits on nothing but their own versions of the forcing conditions. What generality earns, economics must justify — Chapter 13 prices it.

## References and Further Reading

- Regulation (EU) 2023/1542, Annex XIII (battery passport data requirements) and Articles 73–74 (repurposing responsibilities).

- Neubauer, J., and A. Pesaran. “The Ability of Battery Second Use Strategies to Impact Plug-in Electric Vehicle Prices and Serve Utility Energy Storage Applications.” Journal of Power Sources 196, no. 23 (2011): 10351–10358.

- Birkl, C. R., M. R. Roberts, E. McTurk, P. G. Bruce, and D. A. Howey. “Degradation Diagnostics for Lithium Ion Cells.” Journal of Power Sources 341 (2017): 373–386.

- International Electrotechnical Commission. IEC 61400 series: Wind Energy Generation Systems (design and component certification context). Geneva: IEC.

- Nielsen, J. S., and J. D. Sørensen. “On Risk-Based Operation and Maintenance of Offshore Wind Turbine Components.” Reliability Engineering & System Safety 96, no. 1 (2011): 218–229.

- IEEE Standards Association. IEEE 1547-2018 (DER interconnection settings context for §12.4). IEEE, 2018.

- Melin, H. E., et al. “Global Implications of the EU Battery Regulation.” Science 373, no. 6553 (2021): 384–387. The strictest-common-denominator dynamic of Section 12.2.

- International Electrotechnical Commission. IEC 63056 (secondary lithium cells for energy storage) and IEC 62933 series (electrical energy storage systems). Geneva: IEC. The certification context the second-life transaction’s attestations reference.

- DNV. Standard DNV-ST-0262: Lifetime Extension of Wind Turbines. The fatigue-budget assessment practice the load-milestone events feed.